Modular Building Connections: A Review

# Konstantinos Daniel Tsavdaridis1, Dan-Adrian Corfar2

1Professor of Structural Engineering, School of Mathematics, Computer Science and Engineering, City, University of London, London, EC1V 0HB, UK

e-mail: konstantinos.tsavdaridis@city.ac.uk; web page: https://kostasdaniel.blogspot.com/

2PhD Candidate, School of Mathematics, Computer Science and Engineering, City, University of London, London, EC1V 0HB, UK

e-mail: Dan.Corfar@city.ac.uk

Keywords: Inter-Module Connections, Modular Building Systems, Demountability, Reusability, Multi-Attribute Ranking, Damage Control.

Abstract. Recently the development of inter-module connections (IMCs) for steel modular building systems (MBSs) has gained traction with many researchers and engineers being in pursuit of universally performant connection systems. While numerous studies reviewed IMCs for hot-rolled steel MBSs, most of them focused on a limited number of connections and were inconsistent in naming conventions and classification methods, posing a challenge for the development of new and meaningful connections. The present study aims to provide a harmonised overview of the existing literature by proposing a unified nomenclature and a systematic classification based on the method of joining. A multi-attribute ranking system was developed and employed to identify “must-have” features for the development of future designs and key areas of improvement for existing configurations, serving as a useful decision-making tool for both researchers and practitioners concerned with this topic.

# INTRODUCTION

Modular Building Systems (MBSs) are a Modern Method of Construction (MMC) which adopts principles of Offsite Manufacturing (OSM) and Offsite Construction (OSC), delivering ready-to-install, fully finished structural modules; MBSs enable reduced manufacturing costs due to standardisation and mass production, speed of onsite construction guaranteed by straight-forward assembly sequences, and superior end-products delivered by better quality control and improved accuracy of production lines [1]. These innate advantages have bolstered the confidence to adopt  steel MBSs in the high-rise construction sector, where corner-supported modules are typically preferred for the high buckling resistance of hot-rolled steel SHS columns and the planning freedom from unobstructed walls [2].

To date, several studies have investigated the structural behaviour of multi-storey and high-rise steel MBSs [3–7], reaching a consensus that inter-module connections (IMCs) are paramount in the structural performance of steel MBSs. In the past decade, the development of IMCs for hot-rolled steel MBSs has gained traction, putting researchers and engineers in pursuit of universally performant connection systems. This led to the development of a large and growing volume of proposed IMCs which introduced a pressing need for harmonisation of the literature to promote a well-structured development of future designs. While numerous studies reviewed IMCs for hot-rolled steel MBSs, even the most noteworthy of them [8–12] focused on a limited number of connections and were inconsistent in naming conventions and classification methods, posing a challenge for the development of new and meaningful connections.

The present study aims to provide a harmonised overview of the existing literature by proposing a unified nomenclature and a systematic classification based on the method of joining. The main advantages of this classification stem from its consistency all-throughout based on joint typologies and the scalability which supports its adoption and further expansion as new connections are proposed. Moreover, a new multi-attribute ranking system was developed and employed to identify “must-have” features and key areas of improvement for existing connections.

# STATE-OF-THE-ART INTER-MODULE CONNECTIONS

In the present section the wide and often tangled literature of IMCs for hot-rolled steel MBSs was systematically re-organised, drawing attention to the key features of these systems. IMCs were classified and labelled under three main categories, based on the main coupling method: locking devices (LD01-07), post-tensioned (PT01-06), and bolted joints (CTC01-16, BTB01-12, FTF01-19).

## IMCs with locking devices

Seven innovative IMCs were identified in this category (Table 1). In general, these joints used either tool-driven or gravity-actioned mechanisms as showed in Figure 1.

| ID code | Joint typology | ID code | Joint typology |
| --- | --- | --- | --- |
| LD01 [13] | Twistlock | LD05 [14] | Torque-activated pin device |
| LD02 [15] | Interlocking grooves | LD06 [16] | Self-locking spring-activated tabs |
| LD03 [17] | Rotary device and screwed nut | LD07 [18] | Self-locking spring-activated sliding blocks |
| LD04 [19] | Self-locking plug-in device |  |  |

Table 1 : IMCs with locking devices

Figure 1. IMCs with locking devices

## Post-tensioned (PT) IMCs

PT IMCs (Table 2) consisted of rods inserted through hollow sections (continuously throughout the full length of columns). When necessary, bar continuity was ensured by couplers or sleeve nuts, while nuts tightened the rods to connecting plates, which could also enhance the shear force transfer by various types of shear keys (Figure 2).

| ID code | Joint typology | ID code | Joint typology |
| --- | --- | --- | --- |
| PT01 [20] | PT rods through SHS columns | PT04 [21] | PT rods coupled through shear keys |
| PT02 [22] | PT rods clamped through shear keys | PT05 [23] | PT rods coupled through shear keys |
| PT03 [24] | PT rods coupled through steel boxes | PT06 [25] | PT rods through SHS columns |

Table 2 : Post-tensioned IMCs

Figure 2. Post-tensioned IMCs

## Bolted IMCs

Bolted IMCs were the most diverse type of IMCs, using common, easy-to-manufacture steel parts such as corner fittings, plates, bolts, screws, nuts, washers, and other components like pins, tenons, spigots, or rubber layers. Due to the multitude of existing configurations, these connections were further classified based on joint type.

### Column-to-column (CTC) connections

The most common typology of bolted IMCs are CTC joints (Table 3) derived from classic splice joints for tubular sections, with plates which are welded to the ends of the hollow sections and clamped together by bolts (Figure 3).

| ID code | Joint typology | ID code | Joint typology |
| --- | --- | --- | --- |
| CTC01 [26] | Vertical and horizontal plates | CTC09 [27] | Intermediate plates and interior bolts |
| CTC02 [28] | Up-down I beams | CTC10 [29] | Cruciform plate |
| CTC03 [30] | Cover plate with blind bolts | CTC11 [31] | Interlocking pins and connecting plates |
| CTC04 [32] | Bonded sleeve splice joint | CTC12 [33] | Connecting plates and resilient layers |
| CTC05 [34] | Connecting plate and access holes | CTC13 [35] | In-build tenon and side-plates |
| CTC06 [36] | Extended connection plate | CTC14 [37] | Shape-memory alloy (SMA) bolts |
| CTC07 [38] | Bolted intermediate plate | CTC15 [39] | Guiding tenons and internal steel pipes |
| CTC08 [40] | Overlaying extended endplates | CTC16 [41] | Interconnecting tube and nut |

Table 3 : Column-to-columns bolted IMCs

Figure 3. Column-to-column bolted IMCs

### Beam-to-beam (BTB) connections

BTB connections relocate the joint between floor and ceiling beams (Table 4). The common elements employed in this type of connection are gusset plates of various geometries, engineered to closely fill the gaps between modules, optimising the unusable space between each of the floor and ceiling cassettes (Figure 4).

| ID code | Joint typology | ID code | Joint typology |
| --- | --- | --- | --- |
| BTB01 [42] | Cruciform plate | BTB07 [43] | Interpenetrating tenon devices |
| BTB02 [44] | Connector plates | BTB08 [45] | Cross-shaped plug-in connector |
| BTB03 [46] | Steel gusset plates | BTB09 [47] | Bolted plug-in adapter with spring pins |
| BTB04 [22] | Bolted intermediate tenon plate | BTB10 [48] | Bolted gusset plates |
| BTB05 [49] | Cruciform socket-shaped tenon | BTB11 [50] | Plug-in tenon and high-strength and blind bolts |
| BTB06 [51] | Plug-in tenon device | BTB12 [52] | Bolted PFC double beam |

Table 4 : Beam-to-beam bolted IMCs

Figure 4. Beam-to-beam bolted IMCs

### Fitting-to-fitting (FTF) connections

The third type of bolted IMCs stems from the common feature of engaging modules through corner fittings (Table 5). The shape of these cast corners ranges from the classic ISO design adopted from shipping containers to more refined and computationally optimised topologies (Figure 5).

| ID code | Joint typology | ID code | Joint typology |
| --- | --- | --- | --- |
| FTF01 [53] | Steel hollow cube bracket | FTF11 [54] | Square pipe blocks with internal circular pipes |
| FTF02 [55] | Intermediate plate | FTF12 [56] | Gusset plates and locating pins |
| FTF03 [57] | Gusset plates and locating pins | FTF13 [25] | Bolted corner fittings |
| FTF04 [58] | Tie plates and spigots | FTF14 [59] | Vertical bolts through columns |
| FTF05 [60] | Tie plates and twist locks | FTF15 [61] | Corner fittings with internal threaded aperture |
| FTF06 [62] | Rubber isolator and steel clamps | FTF16 [63] | Corner fittings with rubber isolation |
| FTF07 [64] | Transverse clamps | FTF17 [65] | Bolted endplates and intermediate plates |
| FTF08 [64] | Cross-shaped clamp | FTF18 [66] | External bolts and positioning plate |
| FTF09 [64] | X-shaped clamp | FTF19 [67] | Cruciform plate and horizontal bolts |
| FTF10 [64] | Bolted corner fittings |  |  |

Table 5 : Fitting-to-fitting bolted IMCs

Figure 5. Fitting-to-fitting bolted IMCs

# MULTI-ATTRIBUTE RANKING OF IMCS

## The framework of the proposed scoring system

The scoring system (Table 6) was based on the same three core criteria used by Srisangeerthanan et al. [12], namely, structural, manufacturing and constructional performance attributes, while the scale and marking criteria were fully re-configured by adopting a more holistic perspective based on qualitative analysis and engineering intuition. Additionally, characteristics such as design resilience through re-centring and energy dissipation, reusability and design flexibility were not explicitly considered in the metrics of the previous study and were thus included in the present review. Scoring for all attributes was done on 3-point rating scales, except for the vertical and horizontal load transfer (VH) attribute where a 0-1 integer scale was adopted based on whether a connection met or not the criterium. The maximum possible score for a system was 25 points, while the minimum a connection could score was 8.

In lack of consistent quantitative data about the designs considered, a qualitative approach based on visual inspection combined with engineering knowledge and intuition was adopted in the characterisation of the connection systems. Weightings were kept equal to ensure an overall level of impartiality. Nevertheless, it is believed that through the scoring metrics proposed herein a practical, comprehensive, and scalable multi-attribute ranking system was achieved, which could be used as is or tailored by researchers and practising engineers for other specific objectives.

| A. Structural performance metrics |  |  |
| --- | --- | --- |
| Metric | Score | Description |
| VH | 0 | Does not meet the requirement |
|  | 1 | Meets the requirement |
| DR | 1 | Low energy dissipating capability, no self-centring |
|  | 2 | Moderate energy dissipating capability, moderate or no self-centring |
|  | 3 | Good energy dissipating capability, self-centring |
| FD | 1 | Limited scaling opportunities |
|  | 2 | Moderate scaling opportunities |
|  | 3 | Good scaling opportunities |
| B. Constructional performance metrics |  |  |
| Metric | Score | Description |
| DfA | 1 | Complex: no self-aligning/self-locating features, large number of tasks, difficult access, complex tooling |
|  | 2 | Moderate: self-aligning/self-locating features, moderate number of tasks, moderate access, moderate tooling |
|  | 3 | Lean: efficient self-aligning/self-locating features, small number of tasks, easy access, simple tooling |
| DfD | 1 | Difficult to disassemble |
|  | 2 | Easy to disassemble and parts of the assembly need to be replaced |
|  | 3 | Easy to disassemble and parts of the assembly can be reused |
| TC | 1 | Limited tolerance control |
|  | 2 | Moderate tolerance control |
|  | 3 | Good tolerance control |
| C. Manufacturing performance metrics |  |  |
| Metric | Score | Description |
| JC | 1 | Large number of parts; complex geometry; complex manufacturing sequences; difficult to mass-produce; |
|  | 2 | Moderate number of parts; reasonable geometry; moderate manufacturing sequences; reasonable to mass-produce |
|  | 3 | Small number of parts; Easy-to-manufacture geometry; easy to mass-produce; |
| EI | 1 | Difficult integration of connection parts into final joint (e.g., welding of complex parts) |
|  | 2 | Reasonable integration of connection parts into final joint (e.g., fastening) |
|  | 3 | Simple joint configuration (e.g., no post-manufacturing integration required or fast procedures such as inter-locking) |
| EP | 1 | Demanding pre-attachment process (e.g., welding of additional components to finished modules) |
|  | 2 | Reasonable pre-attachment process (e.g., fastening of additional components to finished modules, drilling holes) |
|  | 3 | Easy pre-attachment process (e.g., no additional components required, or connection parts are already integrated into module framing i.e., corner fittings) |
| Notes: VH - Vertical and horizontal connectivity; DR – Design resilience; DF – Design flexibility; DfA – Design for Assembly; DfD – Design for Disassembly; TC – Tolerance control; JC – Joint complexity; EI – Ease of integration; EP – Ease of pre-assembly |  |  |

Table 6 : Description of the proposed scoring system

# APPLICATION OF THE PROPOSED SCORING SYSTEM

The multi-attribute rankings presented herein were used to determine the highest-scoring IMCs for each performance category, revealing the most promising IMCs per attribute as well as the best-performing system for each type of connection.

## Overall ranking

In this section the discussion focused on the highest overall scoring IMCs and their bespoke characteristics, illustrated in Figure 9, while complete data, plots and scoring method for each metric are available in the full review study [68].

Overall, the highest scoring systems were FTF06 and FTF11 joints with totals of 21 points each. FTF06 scored best in metrics such as DR, DF, DfD and TC due to its flexible, resilient, and demountable configuration. The only improvements could be reducing the number of bolts which require on-site tensioning, while manufacturing complexity of the lead-rubber bearing may be mitigated by using a high damping rubber bearing instead.

The next four joints were FTF19, FTF12, FTF03, and BTB09, coming second with totals of 20 points. These joints displayed well-balanced scores for all metrics, demonstrating robust IMCs which could be improved in terms of resilience, reuse opportunities or complexity of fabrication.

Next two IMCs scored 19 points each, achieving medium scores in almost all metrics. The CTC11 system highlighted the efficiency of common and easy to manufacture features such as interlocking pins and bolted endplates, while adding means of energy dissipation and damage control would help improving its resilience and demountability. Connection CTC12 had an efficient configuration made of common parts with good energy dissipation provided by the rubber layers, yet the inclusion of self-aligning features and mitigating the difficulties in repairing or replacing the damaged endplates after an earthquake could be considered for improving its structural and constructional metrics.

In the end, it must be noted that the purpose of the ranking discussed above was not to suggest that one IMC would be better than all the others, as there is no fit-for-all solution in multi-dimensional problems like that of assessing the performance of IMCs. As the discussion demonstrates, the proposed framework is useful in revealing key features which make efficient designs both in terms of structural, constructional, and manufacturing aspects, while it also uncovers areas which require improvement, serving as a good starting point for future research directions.

Figure 9. Overall ranking of IMCs

# concluding remarks

This study provided a harmonised overview of the relevant literature, having reviewed, classified, and indexed sixty IMCs using a unified nomenclature and a systematic and consistent classification system based on joint typology.

A new multi-attribute ranking method was developed using a qualitative approach, holding a certain degree of subjectivity and presented in detail in a complementary paper. Nevertheless, the adoption of the proposed multi-attribute ranking system can facilitate future designs, as well as enhance existing connections in low-scoring areas, serving as a useful decision-making tool for both researchers and practitioners concerned with this topic.

The multi-attribute ranking system revealed that the use of corner fittings, bolted joints, self-aligning/locating parts, and damage control devices were all must-have features for IMCs with all-round performance.

# REFERENCES

[1]	Goodier C, Gibb A. Future opportunities for offsite in the UK. Constr Manag Econ 2007;25:585–95. https://doi.org/10.1080/01446190601071821.

[2]	Lawson RM, Ogden RG, Goodier C. Design in Modular Construction. Oxon: CRC Press; 2014.

[3]	Lacey AW, Chen W, Hao H, Bi K. Structural response of modular buildings – An overview. J Build Eng 2018;16:45–56. https://doi.org/10.1016/j.jobe.2017.12.008.

[4]	Ferdous W, Bai Y, Ngo TD, Manalo A, Mendis P. New advancements, challenges and opportunities of multi-storey modular buildings – A state-of-the-art review. Eng Struct 2019;183:883–93. https://doi.org/10.1016/j.engstruct.2019.01.061.

[5]	Thai H-T, Ngo T, Uy B. A review on modular construction for high-rise buildings. Structures 2020;28:1265–90. https://doi.org/10.1016/j.istruc.2020.09.070.

[6]	Ye Z, Giriunas K, Sezen H, Wu G, Feng D-C. State-of-the-art review and investigation of structural stability in multi-story modular buildings. J Build Eng 2021;33. https://doi.org/10.1016/j.jobe.2020.101844.

[7]	Wang Z, Tsavdaridis KD. Optimality criteria-based minimum-weight design method for modular building systems subjected to generalised stiffness constraints: A comparative study. Eng Struct 2022;251:113472. https://doi.org/10.1016/j.engstruct.2021.113472.

[8]	Deng E-F, Zong L, Ding Y, Zhang Z, Zhang J-F, Shi F-W, et al. Seismic performance of mid-to-high rise modular steel construction - A critical review. Thin-Walled Struct 2020;155:106924. https://doi.org/10.1016/j.tws.2020.106924.

[9]	Chen Z, Khan K, Khan A, Javed K, Liu J. Exploration of the multidirectional stability and response of prefabricated volumetric modular steel structures. J Constr Steel Res 2021;184. https://doi.org/10.1016/j.jcsr.2021.106826.

[10]	Lacey AW, Chen W, Hao H, Bi K. Review of bolted inter-module connections in modular steel buildings. J Build Eng 2019;23:207–19. https://doi.org/10.1016/j.jobe.2019.01.035.

[11]	Li Z, Tsavdaridis KD, Gardner L. A Review of Optimised Additively Manufactured Steel Connections for Modular Building Systems. In: Meboldt M, Klahn C, editors. Ind. Addit. Manuf., Cham: Springer International Publishing; 2021, p. 357–73. https://doi.org/10.1007/978-3-030-54334-1_25.

[12]	Srisangeerthanan S, Hashemi MJ, Rajeev P, Gad E, Fernando S. Review of performance requirements for inter-module connections in multi-story modular buildings. J Build Eng 2020;28. https://doi.org/10.1016/j.jobe.2019.101087.

[13]	BSI. Series 1 freight containers - Handling and securing. London: BSI; 2017.

[14]	Srisangeerthanan S, Hashemi MJ, Rajeev P, Gad E, Fernando S. Fully-Modular Buildings Through a Proposed Inter-module Connection. Icsecm 2019 2021:303–12. https://doi.org/10.1007/978-981-15-7222-7_26.

[15]	Sharafi P, Mortazavi M, Samali B, Ronagh H. Interlocking system for enhancing the integrity of multi-storey modular buildings. Autom Constr 2018;85:263–72. https://doi.org/10.1016/j.autcon.2017.10.023.

[16]	Chen Z, Wang J, Liu J, Khan K. Seismic behavior and moment transfer capacity of an innovative self-locking inter-module connection for modular steel building. Eng Struct 2021;245:112978. https://doi.org/10.1016/j.engstruct.2021.112978.

[17]	Chen Z, Liu Y, Zhong X, Liu J. Rotational stiffness of inter-module connection in mid-rise modular steel buildings. Eng Struct 2019;196. https://doi.org/10.1016/j.engstruct.2019.06.009.

[18]	Liu J, Chen Z, Wang J. Sliding Block Type Modular Building Self-Locking Connection Node (in Chinese). CN 111287331 A, 2020.

[19]	Dai X-M, Zong L, Ding Y, Li Z-X. Experimental study on seismic behavior of a novel plug-in self-lock joint for modular steel construction. Eng Struct 2019;181:143–64. https://doi.org/10.1016/j.engstruct.2018.11.075.

[20]	Farnsworth D. Modular building unit connection system. US9366020B2, 2016.

[21]	Lacey AW, Chen W, Hao H, Bi K, Tallowin FJ. Shear behaviour of post-tensioned inter-module connection for modular steel buildings. J Constr Steel Res 2019;162. https://doi.org/10.1016/j.jcsr.2019.105707.

[22]	Pang SD, Liew JYR, Dai Z, Wang Y. Prefabricated Prefinished Volumetric Construction Joining Techniques Review 2016.

[23]	Chua YS, Liew JYR, Pang SD. Robustness of Prefabricated Prefinished Volumetric Construction (PPVC) High-rise Building, 2018. http://dx.doi.org/10.4995/ASCCS2018.2018.6955.

[24]	Sanches R, Mercan O, Roberts B. Experimental investigations of vertical post-tensioned connection for modular steel structures. Eng Struct 2018;175:776–89. https://doi.org/10.1016/j.engstruct.2018.08.049.

[25]	Epaminondas K. Unitised Building System. AU2016206222B2, 2016.

[26]	Styles AJ, Luo FJ, Bai Y, Murray-Parkes JB. Effects of joint rotational stiffness on structural responses of multi-storey modular buildings. In: Mair RJ, Soga K, Jin Y, Parlikad AK, Schooling JM, editors., ICE Publishing; 2016, p. 457–62. https://doi.org/10.1680/tfitsi.61279.457.

[27]	Wang Y, Xia J, Ma R, Xu B, Wang T. Experimental Study on the Flexural Behavior of an Innovative Modular Steel Building Connection with Installed Bolts in the Columns. Appl Sci 2019;9. https://doi.org/10.3390/app9173468.

[28]	Chen C, Cai YQ, Chiew SP. Finite Element Analysis of Up-down Steel Connectors for Volumetric Modular Construction. 12th Int. Conf. Steel Space Compos. Struct., 2014, p. 173–9.

[29]	Deng E-F, Zong L, Ding Y, Dai X-M, Lou N, Chen Y. Monotonic and cyclic response of bolted connections with welded cover plate for modular steel construction. Eng Struct 2018;167:407–19. https://doi.org/10.1016/j.engstruct.2018.04.028.

[30]	Cho B-H, Lee J-S, Kim H, Kim D-J. Structural Performance of a New Blind-Bolted Frame Modular Beam-Column Connection under Lateral Loading. Appl Sci 2019;9. https://doi.org/10.3390/app9091929.

[31]	Lacey AW, Chen W, Hao H, Bi K. New interlocking inter-module connection for modular steel buildings: Experimental and numerical studies. Eng Struct 2019;198. https://doi.org/10.1016/j.engstruct.2019.109465.

[32]	Qiu C, Bai Y, Zhang L, Jin L. Bending Performance of Splice Connections for Assembly of Tubular Section FRP Members: Experimental and Numerical Study. J Compos Constr 2019;23. https://doi.org/10.1061/(asce)cc.1943-5614.0000964.

[33]	Sendanayake SV, Thambiratnam DP, Perera N, Chan T, Aghdamy S. Seismic mitigation of steel modular building structures through innovative inter-modular connections. Heliyon 2019;5:e02751. https://doi.org/10.1016/j.heliyon.2019.e02751.

[34]	SCI. Building Design Using Modules. Berkshire: SCI; 2007.

[35]	Ma R, Xia J, Chang H, Xu B, Zhang L. Experimental and numerical investigation of mechanical properties on novel modular connections with superimposed beams. Eng Struct 2021;232. https://doi.org/10.1016/j.engstruct.2021.111858.

[36]	Choi K-S, Lee H-C, Kim H-J. Influence of Analytical Models on the Seismic Response of Modular Structures. J Korea Inst Struct Maint Insp 2016;20:74–85. https://doi.org/10.11112/jksmi.2016.20.2.074.

[37]	Sultana P, Youssef MA. Seismic Performance of Modular Steel-Braced Frames Utilizing Superelastic Shape Memory Alloy Bolts in the Vertical Module Connections. J Earthq Eng 2018;24:628–52. https://doi.org/10.1080/13632469.2018.1453394.

[38]	Yang H. Performance analysis of semi-rigid connections in prefabricated high-rise steel structures. Structures 2020;28:837–46. https://doi.org/10.1016/j.istruc.2020.09.036.

[39]	Chain P. Pre-fabricated Pre-finished Volumetric Construction (PPVC) in Singapore: NTU Case Studies (Residential Halls) 2018.

[40]	Gunawardena T. Behaviour of Prefabricated Modular Buildings Subjected to Lateral Loads. The University of Melbourne, 2016.

[41]	O’Brien J. A Structural Module with Vertical Ties. WO2019219286A1, 2019.

[42]	Park K-S, Moon J, Lee S-S, Bae K-W, Roeder CW. Embedded steel column-to-foundation connection for a modular structural system. Eng Struct 2016;110:244–57. https://doi.org/10.1016/j.engstruct.2015.11.034.

[43]	Khan K, Yan J-B. Finite Element Analysis on Seismic Behaviour of Novel Joint in Prefabricated Modular Steel Building. Int J Steel Struct 2020;20:752–65. https://doi.org/10.1007/s13296-020-00320-w.

[44]	Lyu Y-F, Li G-Q, Cao K, Zhai S-Y, Li H, Chen C, et al. Behavior of splice connection during transfer of vertical load in full-scale corner-supported modular building. Eng Struct 2021;230. https://doi.org/10.1016/j.engstruct.2020.111698.

[45]	Zhang G, Xu L-H, Li Z-X. Development and seismic retrofit of an innovative modular steel structure connection using symmetrical self-centering haunch braces. Eng Struct 2021;229. https://doi.org/10.1016/j.engstruct.2020.111671.

[46]	Lee S, Park J, Kwak E, Shon S, Kang C, Choi H. Verification of the Seismic Performance of a Rigidly Connected Modular System Depending on the Shape and Size of the Ceiling Bracket. Mater Basel 2017;10. https://doi.org/10.3390/ma10030263.

[47]	Nadeem G, Safiee NA, Abu Bakar N, Abd Karim I, Mohd Nasir NA. Finite Element Analysis of Proposed Self-Locking Joint for Modular Steel Structures. Appl Sci 2021;11:9277. https://doi.org/10.3390/app11199277.

[48]	Bowron J. Locating Pin Assembly For A Modular Frame. WO2020010463A1, 2020.

[49]	Deng E-F, Yan J-B, Ding Y, Zong L, Li Z-X, Dai X-M. Analytical and numerical studies on steel columns with novel connections in modular construction. Int J Steel Struct 2017;17:1613–26. https://doi.org/10.1007/s13296-017-1226-5.

[50]	Li YM. Mechanical Behavior Study on the New Bolted Joint of Modular Steel Structure. Master Dissertation. Tianjin University, 2019.

[51]	Chen Z, Liu J, Yu Y. Experimental study on interior connections in modular steel buildings. Eng Struct 2017;147:625–38. https://doi.org/10.1016/j.engstruct.2017.06.002.

[52]	Xu B, Xia J, Chang H, Ma R, Zhang L. Flexural behaviour of pairs of laminated unequal channel beams with different interfacial connections in corner-supported modular steel buildings. Thin-Walled Struct 2020;154. https://doi.org/10.1016/j.tws.2020.106792.

[53]	Hwan Doh J, Ho NM, Miller D, Peters T, Carlson D, Lai P. Steel Bracket Connection on Modular Buildings. J Steel Struct Constr 2017;02. https://doi.org/10.4172/2472-0437.1000121.

[54]	Lee J-S, Lee H-D, Shin K-J, Kim H-J, Lee K-M. Structural Performance Evaluation of Modular Connections Using Developed Blocks. Int J Steel Struct 2021. https://doi.org/10.1007/s13296-021-00500-2.

[55]	Yu Y, Chen Z. Rigidity of corrugated plate sidewalls and its effect on the modular structural design. Eng Struct 2018;175:191–200. https://doi.org/10.1016/j.engstruct.2018.08.039.

[56]	Bowron J, Gulliford J, Churchill E, Cerone J, Mallie J. Modular Building Units, And Methods Of Constructing And Transporting Same. WO2014127472A1, 2014.

[57]	Bowron J. Structural Modular Building Connector. US10947716B2, 2021.

[58]	Shan S, Pan W. Structural design of high‐rise buildings using steel‐framed modules: A case study in Hong Kong. Struct Des Tall Spec Build 2020. https://doi.org/10.1002/tal.1788.

[59]	Miller R. Prefabricated Modular Buildings. WO2019023604A1, 2019.

[60]	Shi F, Wang H, Zong L, Ding Y, Su J. Seismic behavior of high-rise modular steel constructions with various module layouts. J Build Eng 2020;31. https://doi.org/10.1016/j.jobe.2020.101396.

[61]	Parkhouse L, Vittadini A, Clark MCJ, Andreatta S, Saby VMC. Connection Node For Modular Building Structures. US10907342B1, 2021.

[62]	Wu CX, Yang Y, Wu CY, Yang T, Xu X. Research on seismic behaviour analysis of shock absorbing structure and connecting joints of container assembly structures. Steel Constr 2019;34:1-8+73.

[63]	Chen Z, Chen S, Zhou T, Liu J. A Rubber Isolation System between Columns of Steel Modular Building (in Chinese). CN 110397176 A, 2019.

[64]	Feng R, Shen L, Yun Q. Seismic performance of multi-story modular box buildings. J Constr Steel Res 2020;168. https://doi.org/10.1016/j.jcsr.2020.106002.

[65]	Chen Z, Zhong X, Liu Y, Liu J, Guo N. Numerical study on modular slab–column steel structure based on simplified integrated floor. Adv Struct Eng 2020;23:1195–208. https://doi.org/10.1177/1369433219891582.

[66]	Hu Q, Lin B, Xie Y, Chen W. Study on Mechanical Performance of Connection jointsIn Novel Modular Steel Structure Buildings. IOP Conf Ser Earth Environ Sci 2020;605:012015. https://doi.org/10.1088/1755-1315/605/1/012015.

[67]	Lian J-Y, Deng E-F, He J-M, Cai L-M, Gao S-C, Zhou J-J. Numerical analysis on seismic performance of corner fitting connection in modular steel building. Structures 2021;33:1659–76. https://doi.org/10.1016/j.istruc.2021.05.044.

[68]	Corfar D-A, Tsavdaridis KD. A comprehensive review and classification of inter-module connections for hot-rolled steel modular building systems. J Build Eng 2022;50:104006. https://doi.org/10.1016/j.jobe.2022.104006.