# Safety assessment of autonomous vehicle in variable operational environment using probabilistic modelling

1st of September

In the past we developed an approach to safety assessment of autonomous vehicle (AV) using probabilistic modelling. The approach taken included two essential components:

- Modelling the road hazards as a source of accidents. Road hazards can be qualified with two parameters: i) intervals of between the road hazards, and ii) the duration of the hazards. Both parameters are assumed random variable and are characterised by corresponding probability distributions. We demonstrated that these distributions can be estimated using public datasets.

- Modelling the quality of perception systems (camera, LiDAR, etc.) used to detect the road hazards and to adjust the AV control accordingly. Quality of the perception systems are more difficult to estimate due to the lack of suitable datasets. In such circumstances the safety assessment can be conducted by deploying “sensitivity analysis”, which is effectively a “what-if” study which establish the impact of perception imperfection of AV safety.

In the previous work we developed a model which relied on a small number of parameters and operated at relatively high level of abstraction. For instance, we disregarded the possibility that road hazards and the quality of the AV perception system may vary with the operating conditions. The model parameters were estimated for a relatively narrow range of operating conditions on a motorway (in Germany) for vehicle driving with a speed between 100 and 130 km/h.

In the current work this limitation is overcome. We now consider the case of an AV which goes through a series of scenarios – oscillating between an “intersection” and “driving freely (e.g., on a motorway)”. We further discriminate between two speed ranges: “high” and “slow”.  Finally, the existence of road hazards is indicated by “breaking” undertaken by the vehicle.

The model otherwise is conceptually very similar to the previous model [ITSC’22]. Essentially, rather than considering “homogenous” operating conditions in the current model we discriminate between 8 different states:

- Free, fast, breaking (F_F_B)

- Free, fast, non-breaking (F_F_NB)

- Free, slow, breaking (F_S_B)

- Free, slow, non-breaking (F_S_NB)

- Intersection, fast, breaking (IS_F_B)

- Intersection, fast, non-breaking (IS_F_NB)

- Intersection, slow, breaking (IS_S_B)

- Intersection, slow, non-breaking (IS_S_NB)

Transitions between these states are possible. We used a dataset … to establish the frequency of the transitions between states and the duration of vehicle spending in each of the 8 states listed above before moving to a new state. The analysis of the times in states is shown in a spreadsheet “rsd_events_new_v3_proc.xlsx”.

Each state is modelled with a separate “atomic” model, which captures the transitions to the remaining 7 other states, using “timed activities”, which are parameterised using the data from the excel file about the distributions of the durations spent in the state prior to a transition to a different state. While the model is generic and timed activities with arbitrary probability distributions can be used, for the illustration we assume that all timed activities are exponentially distributed and estimate the rate of the respective exponential distribution from the data (see the excel file).

The states with “breaking” (i.e., X_Y_B, where X is either F or IS and Y is either “F” or “S”) are considered states with hazards. For these we model in some detail the possibility of perception system failing to detect the road hazard immediately upon entering the respective state. Thus, the atomic models of the “breaking states” are slightly more complex than the atomic models of “non-breaking” states.

Full details of the SAN model are presented below.

The models rely on a number of parameters. Most are estimated from the excel file (the timed activities representing the transitions between the states).

The model relies also on a few parameters characterising the perception system reliability in different states:

- Hazard detection probability upon entering a “breaking state”

- Duration of not detecting the hazard given the hazard exists (i.e. upon entering a “breaking” state). These were derived from the previous work (Lyft dataset)

- Probabilities of accident from different conditions in a breaking state XYB (where X is either “F” of “IS”, Y is either “F” or “S”):

  - Hazard has been detected on time, X_Y_B_O_prob

  - Hazard has not been detected given it exists, MisHazFailXYB_prob

  - Detecting Hazard late (HDLateAcc_XYB_prob)

The model was solved for 4 studies, which defined the same parameters for transitions between states, but used different probabilities of accident and probability of not detecting a hazard given it exists. The results from these studies are presented in Appendix A – D.

| Place Attributes: |  |
| --- | --- |
| Place Names | Initial Markings |
| Accident | 0 |
| CPH_Late_FFB | 0 |
| F_F_B | 1 |
| F_F_B_O | 0 |
| F_F_NB | 0 |
| F_S_B | 0 |
| F_S_NB | 0 |
| IS_F_B | 0 |
| IS_F_NB | 0 |
| IS_S_B | 0 |
| IS_S_NB | 0 |

| Timed Activity: | CorHaz2Acci_FFB |
| --- | --- |
| Distribution Parameters | Rate return(((rate_F_F_B2IS_S_B + rate_F_F_B2F_S_B + rate_F_F_B2_IS_S_NB + rate_F_F_B2IS_F_B + rate_F_F_B2IS_F_NB + rate_F_F_B2F_S_NB + rate_F_F_B2F_F_NB) * SafMonFail_FFB_prob)/(1.0 - SafMonFail_FFB_prob)); |
| Activation Predicate | (none) |
| Reactivation Predicate | (none) |

| Timed Activity: | F_F_B2F_F_NB |
| --- | --- |
| Distribution Parameters | Rate return(rate_F_F_B2F_F_NB); |
| Activation Predicate | (none) |
| Reactivation Predicate | (none) |

| Timed Activity: | F_F_B2F_S_B |
| --- | --- |
| Distribution Parameters | Rate return(rate_F_F_B2F_S_B); |
| Activation Predicate | (none) |
| Reactivation Predicate | (none) |

| Timed Activity: | F_F_B2F_S_NB |
| --- | --- |
| Distribution Parameters | Rate return(rate_F_F_B2F_S_NB); |
| Activation Predicate | (none) |
| Reactivation Predicate | (none) |

| Timed Activity: | F_F_B2IS_F_B |
| --- | --- |
| Distribution Parameters | Rate return(rate_F_F_B2IS_F_B); |
| Activation Predicate | (none) |
| Reactivation Predicate | (none) |

| Timed Activity: | F_F_B2IS_F_NB |
| --- | --- |
| Distribution Parameters | Rate return(rate_F_F_B2IS_F_NB); |
| Activation Predicate | (none) |
| Reactivation Predicate | (none) |

| Timed Activity: | F_F_B2IS_S_B |
| --- | --- |
| Distribution Parameters | Rate return(rate_F_F_B2IS_S_B); |
| Activation Predicate | (none) |
| Reactivation Predicate | (none) |

| Timed Activity: | F_F_B2_IS_S_NB |
| --- | --- |
| Distribution Parameters | Rate return(rate_F_F_B2_IS_S_NB); |
| Activation Predicate | (none) |
| Reactivation Predicate | (none) |

| Timed Activity: | HDLateAcc_FFB |
| --- | --- |
| Distribution Parameters | Rate return(1.0/(1.0/rate_F_F_B2F_F_NB - 1.0/OH2OKRate_FFB)); |
| Activation Predicate | (none) |
| Reactivation Predicate | (none) |
| Case Distributions | case 1 return(HDLateAcc_FFB_prob); case 2 //return(rate_F_F_B2F_F_NB/(CH2AccRate_FFB + rate_F_F_B2F_F_NB)); return(1.0 - HDLateAcc_FFB_prob); |

| Timed Activity: | sojournTime |
| --- | --- |
| Distribution Parameters | Rate return(OH2OKRate_FFB); |
| Activation Predicate | (none) |
| Reactivation Predicate | (none) |
| Case Distributions | case 1 return(MisHazFailFFB_prob); case 2 return(1.0 - MisHazFailFFB_prob); |

Model: State_F_F_NB

| Place Attributes: |  |
| --- | --- |
| Place Names | Initial Markings |
| F_F_B | 1 |
| F_F_B_O | 0 |
| F_F_NB | 0 |
| F_S_B | 0 |
| F_S_B_O | 0 |
| F_S_NB | 0 |
| IS_F_B | 0 |
| IS_F_B_O | 0 |
| IS_F_NB | 0 |
| IS_S_B | 0 |
| IS_S_B_O | 0 |
| IS_S_NB | 0 |

| Timed Activity: | F_F_NB2F_F_B |
| --- | --- |
| Distribution Parameters | Rate return(rate_F_F_NB2F_F_B); |
| Activation Predicate | (none) |
| Reactivation Predicate | (none) |
| Case Distributions | case 1 return(1.0 - F_F_B_O_prob); case 2 return(F_F_B_O_prob); |

| Timed Activity: | F_F_NB2F_S_B |
| --- | --- |
| Distribution Parameters | Rate return(rate_F_F_NB2F_S_B); |
| Activation Predicate | (none) |
| Reactivation Predicate | (none) |
| Case Distributions | case 1 return(1.0 - F_S_B_O_prob); case 2 return(F_S_B_O_prob); |

| Timed Activity: | F_F_NB2F_S_NB |
| --- | --- |
| Distribution Parameters | Rate return(rate_F_F_NB2F_S_NB); |
| Activation Predicate | (none) |
| Reactivation Predicate | (none) |

| Timed Activity: | F_F_NB2IS_F_B |
| --- | --- |
| Distribution Parameters | Rate return(rate_F_F_NB2IS_F_B); |
| Activation Predicate | (none) |
| Reactivation Predicate | (none) |
| Case Distributions | case 1 return(1.0 - IS_F_B_O_prob); case 2 return(IS_F_B_O_prob); |

| Timed Activity: | F_F_NB2IS_F_NB |
| --- | --- |
| Distribution Parameters | Rate return(rate_F_F_NB2IS_F_NB); |
| Activation Predicate | (none) |
| Reactivation Predicate | (none) |

| Timed Activity: | F_F_NB2IS_S_B |
| --- | --- |
| Distribution Parameters | Rate return(rate_F_F_NB2IS_S_B); |
| Activation Predicate | (none) |
| Reactivation Predicate | (none) |
| Case Distributions | case 1 return(1.0 - IS_S_B_O_prob); case 2 return(IS_S_B_O_prob); |

| Timed Activity: | F_F_NB2IS_S_NB |
| --- | --- |
| Distribution Parameters | Rate return(rate_F_F_NB2IS_S_NB); |
| Activation Predicate | (none) |
| Reactivation Predicate | (none) |

Model: State_F_S_B

| Place Attributes: |  |
| --- | --- |
| Place Names | Initial Markings |
| Accident | 0 |
| CPH_Late_FSB | 0 |
| F_F_B | 1 |
| F_F_NB | 0 |
| F_S_B | 0 |
| F_S_B_O | 0 |
| F_S_NB | 0 |
| IS_F_B | 0 |
| IS_F_NB | 0 |
| IS_S_B | 0 |
| IS_S_NB | 0 |

| Timed Activity: | CorHaz2Acci_FSB |
| --- | --- |
| Distribution Parameters | Rate return((rate_F_S_B2IS_S_B + rate_F_S_B2IS_S_NB + rate_F_S_B2F_S_NB + rate_F_S_B2IS_F_B + rate_F_S_B2IS_F_NB + rate_F_S_B2F_F_NB + rate_F_S_B2F_F_B) * SafMonFailFSB_prob / (1.0 - SafMonFailFSB_prob)); |
| Activation Predicate | (none) |
| Reactivation Predicate | (none) |

| Timed Activity: | F_S_B2F_F_B |
| --- | --- |
| Distribution Parameters | Rate return(rate_F_S_B2F_F_B); |
| Activation Predicate | (none) |
| Reactivation Predicate | (none) |

| Timed Activity: | F_S_B2F_F_NB |
| --- | --- |
| Distribution Parameters | Rate return(rate_F_S_B2F_F_NB); |
| Activation Predicate | (none) |
| Reactivation Predicate | (none) |

| Timed Activity: | F_S_B2F_S_NB |
| --- | --- |
| Distribution Parameters | Rate return(rate_F_S_B2F_S_NB); |
| Activation Predicate | (none) |
| Reactivation Predicate | (none) |

| Timed Activity: | F_S_B2IS_F_B |
| --- | --- |
| Distribution Parameters | Rate return(rate_F_S_B2IS_F_B); |
| Activation Predicate | (none) |
| Reactivation Predicate | (none) |

| Timed Activity: | F_S_B2IS_F_NB |
| --- | --- |
| Distribution Parameters | Rate return(rate_F_S_B2IS_F_NB); |
| Activation Predicate | (none) |
| Reactivation Predicate | (none) |

| Timed Activity: | F_S_B2IS_S_B |
| --- | --- |
| Distribution Parameters | Rate return(rate_F_S_B2IS_S_B); |
| Activation Predicate | (none) |
| Reactivation Predicate | (none) |

| Timed Activity: | F_S_B2IS_S_NB |
| --- | --- |
| Distribution Parameters | Rate return(rate_F_S_B2IS_S_NB); |
| Activation Predicate | (none) |
| Reactivation Predicate | (none) |

| Timed Activity: | HDLateAcc_FSB |
| --- | --- |
| Distribution Parameters | Rate return(1.0/(1.0/rate_F_S_B2F_F_NB - 1.0/OH2OKRate_FSB)); |
| Activation Predicate | (none) |
| Reactivation Predicate | (none) |
| Case Distributions | case 1 return(HDLateAcc_FSB_prob); case 2 return(1.0 - HDLateAcc_FSB_prob); |

| Timed Activity: | sojournTime |
| --- | --- |
| Distribution Parameters | Rate return(OH2OKRate_FSB); |
| Activation Predicate | (none) |
| Reactivation Predicate | (none) |
| Case Distributions | case 1 return(MisHazFailFSB_prob); case 2 return(1.0 - MisHazFailFSB_prob); |

Model: State_F_S_NB

| Place Attributes: |  |
| --- | --- |
| Place Names | Initial Markings |
| F_F_B | 1 |
| F_F_B_O | 0 |
| F_F_NB | 0 |
| F_S_B | 0 |
| F_S_B_O | 0 |
| F_S_NB | 0 |
| IS_F_B | 0 |
| IS_F_B_O | 0 |
| IS_F_NB | 0 |
| IS_S_B | 0 |
| IS_S_B_O | 0 |
| IS_S_NB | 0 |

| Timed Activity: | F_S_NB2F_F_B |
| --- | --- |
| Distribution Parameters | Rate return(rate_F_S_NB2F_F_B); |
| Activation Predicate | (none) |
| Reactivation Predicate | (none) |
| Case Distributions | case 1 return(1.0 - F_F_B_O_prob); case 2 return(F_F_B_O_prob); |

| Timed Activity: | F_S_NB2F_F_NB |
| --- | --- |
| Distribution Parameters | Rate return(rate_F_S_NB2F_F_NB); |
| Activation Predicate | (none) |
| Reactivation Predicate | (none) |

| Timed Activity: | F_S_NB2F_S_B |
| --- | --- |
| Distribution Parameters | Rate return(rate_F_S_NB2F_S_B); |
| Activation Predicate | (none) |
| Reactivation Predicate | (none) |
| Case Distributions | case 1 return(1.0 - F_S_B_O_prob); case 2 return(F_S_B_O_prob); |

| Timed Activity: | F_S_NB2IS_F_B |
| --- | --- |
| Distribution Parameters | Rate return(rate_F_S_NB2IS_F_B); |
| Activation Predicate | (none) |
| Reactivation Predicate | (none) |
| Case Distributions | case 1 return(1.0 - IS_F_B_O_prob); case 2 return(IS_F_B_O_prob); |

| Timed Activity: | F_S_NB2IS_F_NB |
| --- | --- |
| Distribution Parameters | Rate return(rate_F_S_NB2IS_F_NB); |
| Activation Predicate | (none) |
| Reactivation Predicate | (none) |

| Timed Activity: | F_S_NB2IS_S_B |
| --- | --- |
| Distribution Parameters | Rate return(rate_F_S_NB2IS_S_B); |
| Activation Predicate | (none) |
| Reactivation Predicate | (none) |
| Case Distributions | case 1 return(1.0 - IS_S_B_O_prob); case 2 return(IS_S_B_O_prob); |

| Timed Activity: | F_S_NB2IS_S_NB |
| --- | --- |
| Distribution Parameters | Rate return(rate_F_S_NB2IS_S_NB); |
| Activation Predicate | (none) |
| Reactivation Predicate | (none) |

Model: State_IS_F_B

| Place Attributes: |  |
| --- | --- |
| Place Names | Initial Markings |
| Accident | 0 |
| CPH_Late_ISSB | 0 |
| F_F_B | 1 |
| F_F_NB | 0 |
| F_S_B | 0 |
| F_S_NB | 0 |
| IS_F_B | 0 |
| IS_F_B_O | 0 |
| IS_F_NB | 0 |
| IS_S_B | 0 |
| IS_S_NB | 0 |

| Timed Activity: | CorHaz2Acci_ISFB |
| --- | --- |
| Distribution Parameters | Rate return((rate_IS_F_B2IS_S_B + rate_IS_F_B2F_S_B + rate_IS_F_B2IS_S_NB + rate_IS_F_B2IS_F_NB + rate_IS_F_B2F_S_NB + rate_IS_F_B2F_F_B + rate_IS_F_B2F_F_NB) * SafMonFailISFB_prob/ (1.0 - SafMonFailISFB_prob)); |
| Activation Predicate | (none) |
| Reactivation Predicate | (none) |

| Timed Activity: | HDLateAcc_ISSB |
| --- | --- |
| Distribution Parameters | Rate return(1.0/(1.0/rate_IS_F_B2IS_S_NB - 1.0/OH2OKRate_ISFB)); |
| Activation Predicate | (none) |
| Reactivation Predicate | (none) |
| Case Distributions | case 1 return(HDLateAcc_ISFB_prob); case 2 return(1.0 - HDLateAcc_ISFB_prob); |

| Timed Activity: | IS_F_B2F_F_B |
| --- | --- |
| Distribution Parameters | Rate return(rate_IS_F_B2F_F_B); |
| Activation Predicate | (none) |
| Reactivation Predicate | (none) |

| Timed Activity: | IS_F_B2F_F_NB |
| --- | --- |
| Distribution Parameters | Rate return(rate_IS_F_B2F_F_NB); |
| Activation Predicate | (none) |
| Reactivation Predicate | (none) |

| Timed Activity: | IS_F_B2F_S_B |
| --- | --- |
| Distribution Parameters | Rate return(rate_IS_F_B2F_S_B); |
| Activation Predicate | (none) |
| Reactivation Predicate | (none) |

| Timed Activity: | IS_F_B2F_S_NB |
| --- | --- |
| Distribution Parameters | Rate return(rate_IS_F_B2F_S_NB); |
| Activation Predicate | (none) |
| Reactivation Predicate | (none) |

| Timed Activity: | IS_F_B2IS_F_NB |
| --- | --- |
| Distribution Parameters | Rate return(rate_IS_F_B2IS_F_NB); |
| Activation Predicate | (none) |
| Reactivation Predicate | (none) |

| Timed Activity: | IS_F_B2IS_S_B |
| --- | --- |
| Distribution Parameters | Rate return(rate_IS_F_B2IS_S_B); |
| Activation Predicate | (none) |
| Reactivation Predicate | (none) |

| Timed Activity: | IS_F_B2IS_S_NB |
| --- | --- |
| Distribution Parameters | Rate return(rate_IS_F_B2IS_S_NB); |
| Activation Predicate | (none) |
| Reactivation Predicate | (none) |

| Timed Activity: | sojournTime |
| --- | --- |
| Distribution Parameters | Rate return(OH2OKRate_ISFB); |
| Activation Predicate | (none) |
| Reactivation Predicate | (none) |
| Case Distributions | case 1 return(MisHazFailISFB_prob); case 2 return(1.0 - MisHazFailISFB_prob); |

Model: State_IS_F_NB

| Place Attributes: |  |
| --- | --- |
| Place Names | Initial Markings |
| F_F_B | 1 |
| F_F_B_O | 0 |
| F_F_NB | 0 |
| F_S_B | 0 |
| F_S_B_O | 0 |
| F_S_NB | 0 |
| IS_F_B | 0 |
| IS_F_B_O | 0 |
| IS_F_NB | 0 |
| IS_S_B | 0 |
| IS_S_B_O | 0 |
| IS_S_NB | 0 |

| Timed Activity: | IS_F_NB2F_F_B |
| --- | --- |
| Distribution Parameters | Rate return(rate_IS_F_NB2F_F_B); |
| Activation Predicate | (none) |
| Reactivation Predicate | (none) |
| Case Distributions | case 1 return(1.0 - F_F_B_O_prob); case 2 return(F_F_B_O_prob); |

| Timed Activity: | IS_F_NB2F_F_NB |
| --- | --- |
| Distribution Parameters | Rate return(rate_IS_F_NB2F_F_NB); |
| Activation Predicate | (none) |
| Reactivation Predicate | (none) |

| Timed Activity: | IS_F_NB2F_S_B |
| --- | --- |
| Distribution Parameters | Rate return(rate_IS_F_NB2F_S_B); |
| Activation Predicate | (none) |
| Reactivation Predicate | (none) |
| Case Distributions | case 1 return(1.0 - F_S_B_O_prob); case 2 return(F_S_B_O_prob); |

| Timed Activity: | IS_F_NB2F_S_NB |
| --- | --- |
| Distribution Parameters | Rate return(rate_IS_F_NB2F_S_NB); |
| Activation Predicate | (none) |
| Reactivation Predicate | (none) |

| Timed Activity: | IS_F_NB2IS_F_B |
| --- | --- |
| Distribution Parameters | Rate return(rate_IS_F_NB2IS_F_B); |
| Activation Predicate | (none) |
| Reactivation Predicate | (none) |
| Case Distributions | case 1 return(1.0 - IS_F_B_O_prob); case 2 return(1.0 - IS_F_B_O_prob); |

| Timed Activity: | IS_F_NB2IS_S_B |
| --- | --- |
| Distribution Parameters | Rate return(rate_IS_F_NB2IS_S_B); |
| Activation Predicate | (none) |
| Reactivation Predicate | (none) |
| Case Distributions | case 1 return(1.0 - IS_S_B_O_prob); case 2 return(IS_S_B_O_prob); |

| Timed Activity: | IS_F_NB2IS_S_NB |
| --- | --- |
| Distribution Parameters | Rate return(rate_IS_F_NB2IS_S_NB); |
| Activation Predicate | (none) |
| Reactivation Predicate | (none) |

Model: State_IS_S_B

| Place Attributes: |  |
| --- | --- |
| Place Names | Initial Markings |
| Accident | 0 |
| CPH_Late_ISSB | 0 |
| F_F_B | 1 |
| F_F_NB | 0 |
| F_S_B | 0 |
| F_S_NB | 0 |
| IS_F_B | 0 |
| IS_F_NB | 0 |
| IS_S_B | 0 |
| IS_S_B_O | 0 |
| IS_S_NB | 0 |

| Timed Activity: | CorHaz2Acci_ISSB |
| --- | --- |
| Distribution Parameters | Rate return((rate_IS_S_B2F_S_B + rate_IS_S_B2IS_S_NB + rate_IS_S_B2IS_F_B + rate_IS_S_B2IS_F_NB + rate_IS_S_B2F_S_NB + rate_IS_S_B2F_F_B + rate_IS_S_B2F_F_NB) * SafMonFailISSB_prob/ (1.0 - SafMonFailISSB_prob)); |
| Activation Predicate | (none) |
| Reactivation Predicate | (none) |

| Timed Activity: | HDLateAcc_ISSB |
| --- | --- |
| Distribution Parameters | Rate return(1.0/(1.0/rate_IS_S_B2IS_S_NB - 1.0/OH2OKRate_ISSB)); |
| Activation Predicate | (none) |
| Reactivation Predicate | (none) |
| Case Distributions | case 1 return(HDLateAcc_ISSB_prob); case 2 return(1.0 - HDLateAcc_ISSB_prob); |

| Timed Activity: | IS_S_B2F_F_B |
| --- | --- |
| Distribution Parameters | Rate return(rate_IS_S_B2F_F_B); |
| Activation Predicate | (none) |
| Reactivation Predicate | (none) |

| Timed Activity: | IS_S_B2F_F_NB |
| --- | --- |
| Distribution Parameters | Rate return(rate_IS_S_B2F_F_NB); |
| Activation Predicate | (none) |
| Reactivation Predicate | (none) |

| Timed Activity: | IS_S_B2F_S_B |
| --- | --- |
| Distribution Parameters | Rate return(rate_IS_S_B2F_S_B); |
| Activation Predicate | (none) |
| Reactivation Predicate | (none) |

| Timed Activity: | IS_S_B2F_S_NB |
| --- | --- |
| Distribution Parameters | Rate return(rate_IS_S_B2F_S_NB); |
| Activation Predicate | (none) |
| Reactivation Predicate | (none) |

| Timed Activity: | IS_S_B2IS_F_B |
| --- | --- |
| Distribution Parameters | Rate return(rate_IS_S_B2IS_F_B); |
| Activation Predicate | (none) |
| Reactivation Predicate | (none) |

| Timed Activity: | IS_S_B2IS_F_NB |
| --- | --- |
| Distribution Parameters | Rate return(rate_IS_S_B2IS_F_NB); |
| Activation Predicate | (none) |
| Reactivation Predicate | (none) |

| Timed Activity: | IS_S_B2IS_S_NB |
| --- | --- |
| Distribution Parameters | Rate return(rate_IS_S_B2IS_S_NB); |
| Activation Predicate | (none) |
| Reactivation Predicate | (none) |

| Timed Activity: | sojournTime |
| --- | --- |
| Distribution Parameters | Rate return(OH2OKRate_ISSB); |
| Activation Predicate | (none) |
| Reactivation Predicate | (none) |
| Case Distributions | case 1 return(MisHazFailISSB_prob); case 2 return(1.0 - MisHazFailISSB_prob); |

Model: State_IS_S_NB

| Place Attributes: |  |
| --- | --- |
| Place Names | Initial Markings |
| F_F_B | 1 |
| F_F_B_O | 0 |
| F_F_NB | 0 |
| F_S_B | 0 |
| F_S_B_O | 0 |
| F_S_NB | 0 |
| IS_F_B | 0 |
| IS_F_B_O | 0 |
| IS_F_NB | 0 |
| IS_S_B | 0 |
| IS_S_B_O | 0 |
| IS_S_NB | 0 |

| Timed Activity: | IS_S_NB2F_F_B |
| --- | --- |
| Distribution Parameters | Rate return(rate_IS_S_NB2F_F_B); |
| Activation Predicate | (none) |
| Reactivation Predicate | (none) |
| Case Distributions | case 1 return(1.0 - F_F_B_O_prob); case 2 return(F_F_B_O_prob); |

| Timed Activity: | IS_S_NB2F_F_NB |
| --- | --- |
| Distribution Parameters | Rate return(rate_IS_S_NB2F_F_NB); |
| Activation Predicate | (none) |
| Reactivation Predicate | (none) |

| Timed Activity: | IS_S_NB2F_S_B |
| --- | --- |
| Distribution Parameters | Rate return(rate_IS_S_NB2F_S_B); |
| Activation Predicate | (none) |
| Reactivation Predicate | (none) |
| Case Distributions | case 1 return(1.0 - F_S_B_O_prob); case 2 return(F_S_B_O_prob); |

| Timed Activity: | IS_S_NB2F_S_NB |
| --- | --- |
| Distribution Parameters | Rate return(rate_IS_S_NB2F_S_NB); |
| Activation Predicate | (none) |
| Reactivation Predicate | (none) |

| Timed Activity: | IS_S_NB2IS_F_B |
| --- | --- |
| Distribution Parameters | Rate return(rate_IS_S_NB2IS_F_B); |
| Activation Predicate | (none) |
| Reactivation Predicate | (none) |
| Case Distributions | case 1 return(1.0 - IS_F_B_O_prob); case 2 return(IS_F_B_O_prob); |

| Timed Activity: | IS_S_NB2IS_F_NB |
| --- | --- |
| Distribution Parameters | Rate return(rate_IS_S_NB2IS_F_NB); |
| Activation Predicate | (none) |
| Reactivation Predicate | (none) |

| Timed Activity: | IS_S_NB2IS_S_B |
| --- | --- |
| Distribution Parameters | Rate return(rate_IS_S_NB2IS_S_B); |
| Activation Predicate | (none) |
| Reactivation Predicate | (none) |
| Case Distributions | case 1 return(1.0 - IS_S_B_O_prob); case 2 return(IS_S_B_O_prob); |

Model: System

| Join Node: FullModel a: |  |
| --- | --- |
| State Variable Name | Submodel Variables |
| Accident | State_IS_S_B->Accident |
|  | State_IS_F_B->Accident |
|  | State_F_S_B->Accident |
|  | State_F_F_B->Accident |
| F_F_B | State_IS_S_B->F_F_B |
|  | State_IS_S_NB->F_F_B |
|  | State_IS_F_B->F_F_B |
|  | State_IS_F_NB->F_F_B |
|  | State_F_S_B->F_F_B |
|  | State_F_S_NB->F_F_B |
|  | State_F_F_B->F_F_B |
|  | State_F_F_NB->F_F_B |
| F_F_B_O | State_IS_S_NB->F_F_B_O |
|  | State_IS_F_NB->F_F_B_O |
|  | State_F_S_NB->F_F_B_O |
|  | State_F_F_B->F_F_B_O |
|  | State_F_F_NB->F_F_B_O |
| F_F_NB | State_IS_S_B->F_F_NB |
|  | State_IS_S_NB->F_F_NB |
|  | State_IS_F_B->F_F_NB |
|  | State_IS_F_NB->F_F_NB |
|  | State_F_S_B->F_F_NB |
|  | State_F_S_NB->F_F_NB |
|  | State_F_F_B->F_F_NB |
|  | State_F_F_NB->F_F_NB |
| F_S_B | State_IS_S_B->F_S_B |
|  | State_IS_S_NB->F_S_B |
|  | State_IS_F_B->F_S_B |
|  | State_IS_F_NB->F_S_B |
|  | State_F_S_B->F_S_B |
|  | State_F_S_NB->F_S_B |
|  | State_F_F_B->F_S_B |
|  | State_F_F_NB->F_S_B |
| F_S_B_O | State_IS_S_NB->F_S_B_O |
|  | State_IS_F_NB->F_S_B_O |
|  | State_F_S_B->F_S_B_O |
|  | State_F_S_NB->F_S_B_O |
|  | State_F_F_NB->F_S_B_O |
| F_S_NB | State_IS_S_B->F_S_NB |
|  | State_IS_S_NB->F_S_NB |
|  | State_IS_F_B->F_S_NB |
|  | State_IS_F_NB->F_S_NB |
|  | State_F_S_B->F_S_NB |
|  | State_F_S_NB->F_S_NB |
|  | State_F_F_B->F_S_NB |
|  | State_F_F_NB->F_S_NB |
| IS_F_B | State_IS_S_B->IS_F_B |
|  | State_IS_S_NB->IS_F_B |
|  | State_IS_F_B->IS_F_B |
|  | State_IS_F_NB->IS_F_B |
|  | State_F_S_B->IS_F_B |
|  | State_F_S_NB->IS_F_B |
|  | State_F_F_B->IS_F_B |
|  | State_F_F_NB->IS_F_B |
| IS_F_B_O | State_IS_S_NB->IS_F_B_O |
|  | State_IS_F_B->IS_F_B_O |
|  | State_IS_F_NB->IS_F_B_O |
|  | State_F_S_NB->IS_F_B_O |

| Join Node: FullModel b: |  |
| --- | --- |
| State Variable Name | Submodel Variables |
| IS_F_B_O | State_F_F_NB->IS_F_B_O |
| IS_F_NB | State_IS_S_B->IS_F_NB |
|  | State_IS_S_NB->IS_F_NB |
|  | State_IS_F_B->IS_F_NB |
|  | State_IS_F_NB->IS_F_NB |
|  | State_F_S_B->IS_F_NB |
|  | State_F_S_NB->IS_F_NB |
|  | State_F_F_B->IS_F_NB |
|  | State_F_F_NB->IS_F_NB |
| IS_S_B | State_IS_S_B->IS_S_B |
|  | State_IS_S_NB->IS_S_B |
|  | State_IS_F_B->IS_S_B |
|  | State_IS_F_NB->IS_S_B |
|  | State_F_S_B->IS_S_B |
|  | State_F_S_NB->IS_S_B |
|  | State_F_F_B->IS_S_B |
|  | State_F_F_NB->IS_S_B |
| IS_S_B_O | State_IS_S_B->IS_S_B_O |
|  | State_IS_S_NB->IS_S_B_O |
|  | State_IS_F_NB->IS_S_B_O |
|  | State_F_S_NB->IS_S_B_O |
|  | State_F_F_NB->IS_S_B_O |
| IS_S_NB | State_IS_S_B->IS_S_NB |
|  | State_IS_S_NB->IS_S_NB |
|  | State_IS_F_B->IS_S_NB |
|  | State_IS_F_NB->IS_S_NB |
|  | State_F_S_B->IS_S_NB |
|  | State_F_S_NB->IS_S_NB |
|  | State_F_F_B->IS_S_NB |
|  | State_F_F_NB->IS_S_NB |

| Performance Variable Model: trans_data |  |  |
| --- | --- | --- |
| Top Level Model Information | Child Model Name | System |
|  | Model Type | Rep/Join |

| Performance Variable : TimeToAccident |  |  |  |
| --- | --- | --- | --- |
| Affecting Models | State_IS_S_B |  |  |
| Impulse Functions |  |  |  |
| Reward Function | (Reward is over all Available Models) if(State_IS_S_B->Accident->Mark() > 0) return(1); else return(0); |  |  |
| Simulator Statistics | Type | Instant of Time |  |
|  | Options | Estimate Mean |  |
|  |  | Estimate Variance |  |
|  |  | Include Lower Bound on Interval Estimate |  |
|  |  | Include Upper Bound on Interval Estimate |  |
|  |  | Estimate Distribution |  |
|  |  | Estimate out of Range Probabilities |  |
|  |  | Confidence Level is Relative |  |
|  | Parameters | Start Time | 100.0,1100.0,2100.0,3100.0,4100.0,5100.0,6100.0,7100.0,8100.0,9100.0, |
|  | Distributions | Lower Bound on Distribution Estimate | 0.0 |
|  |  | Upper Bound on Distribution Estimate | 1.0 |
|  |  | Step Size on Distribution Estimate | 0.1 |
|  | Confidence | Confidence Level | 0.95 |
|  |  | Confidence Interval | 0.05 |

| Range Study Variable Assignments for Study range_st in Project AV_Intersection_with_Motorway a: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Variable | Type | Range Type | Range | Increment | Increment Type | Function | n |
| CH2AccRate_FFB | double | Fixed | 0.0856 | - | - | - | - |
| CH2AccRate_FSB | double | Fixed | 0.0856 | - | - | - | - |
| CH2AccRate_ISFB | double | Fixed | 0.0856 | - | - | - | - |
| CH2AccRate_ISSB | double | Fixed | 0.0856 | - | - | - | - |
| F_F_B_O_prob | double | Fixed | 0.0 | - | - | - | - |
| F_S_B_O_prob | double | Fixed | 0.0 | - | - | - | - |
| IS_F_B_O_prob | double | Fixed | 0.0 | - | - | - | - |
| IS_S_B_O_prob | double | Fixed | 0.0 | - | - | - | - |
| MH2FFB_AccRate | double | Fixed | 0.1 | - | - | - | - |
| MH2FSB_AccRate | double | Fixed | 0.1 | - | - | - | - |
| MH2ISFB_AccRate | double | Fixed | 0.1 | - | - | - | - |
| MH2ISSB_AccRate | double | Fixed | 0.1 | - | - | - | - |
| OH2CH_FFB_scale | double | Fixed | 1.0 | - | - | - | - |
| OH2CH_FSB_scale | double | Fixed | 1.0 | - | - | - | - |
| OH2CH_ISFB_scale | double | Fixed | 1.0 | - | - | - | - |
| OH2CH_ISSB_scale | double | Fixed | 1.0 | - | - | - | - |
| OH2OKRate_FFB | double | Fixed | 4500.0 | - | - | - | - |
| OH2OKRate_FSB | double | Fixed | 2250.0 | - | - | - | - |
| OH2OKRate_ISFB | double | Fixed | 2250.0 | - | - | - | - |
| OH2OKRate_ISSB | double | Fixed | 1225.0 | - | - | - | - |
| rate_F_F_B2F_F_NB | double | Fixed | 609.87 | - | - | - | - |
| rate_F_F_B2F_S_B | double | Fixed | 676.71 | - | - | - | - |
| rate_F_F_B2F_S_NB | double | Fixed | 319.02 | - | - | - | - |
| rate_F_F_B2IS_F_B | double | Fixed | 616.29 | - | - | - | - |
| rate_F_F_B2IS_F_NB | double | Fixed | 551.1 | - | - | - | - |
| rate_F_F_B2IS_S_B | double | Fixed | 549.09 | - | - | - | - |
| rate_F_F_B2_IS_S_NB | double | Fixed | 310.79 | - | - | - | - |
| rate_F_F_NB2F_F_B | double | Fixed | 170.61 | - | - | - | - |
| rate_F_F_NB2F_S_B | double | Fixed | 252.79 | - | - | - | - |
| rate_F_F_NB2F_S_NB | double | Fixed | 361.58 | - | - | - | - |
| rate_F_F_NB2IS_F_B | double | Fixed | 209.77 | - | - | - | - |
| rate_F_F_NB2IS_F_NB | double | Fixed | 202.345 | - | - | - | - |
| rate_F_F_NB2IS_S_B | double | Fixed | 361.87 | - | - | - | - |
| rate_F_F_NB2IS_S_NB | double | Fixed | 273.315 | - | - | - | - |
| rate_F_S_B2F_F_B | double | Fixed | 1080.0 | - | - | - | - |
| rate_F_S_B2F_F_NB | double | Fixed | 604.64 | - | - | - | - |
| rate_F_S_B2F_S_NB | double | Fixed | 465.98 | - | - | - | - |
| rate_F_S_B2IS_F_B | double | Fixed | 0.0 | - | - | - | - |
| rate_F_S_B2IS_F_NB | double | Fixed | 439.22 | - | - | - | - |
| rate_F_S_B2IS_S_B | double | Fixed | 940.01 | - | - | - | - |
| rate_F_S_B2IS_S_NB | double | Fixed | 438.63 | - | - | - | - |
| rate_F_S_NB2F_F_B | double | Fixed | 568.42 | - | - | - | - |
| rate_F_S_NB2F_F_NB | double | Fixed | 223.71 | - | - | - | - |
| rate_F_S_NB2F_S_B | double | Fixed | 355.8 | - | - | - | - |
| rate_F_S_NB2IS_F_B | double | Fixed | 0.0 | - | - | - | - |
| rate_F_S_NB2IS_F_NB | double | Fixed | 264.45 | - | - | - | - |
| rate_F_S_NB2IS_S_B | double | Fixed | 219.02 | - | - | - | - |
| rate_F_S_NB2IS_S_NB | double | Fixed | 228.71 | - | - | - | - |
| rate_IS_F_B2F_F_B | double | Fixed | 986.54 | - | - | - | - |
| rate_IS_F_B2F_F_NB | double | Fixed | 822.71 | - | - | - | - |
| rate_IS_F_B2F_S_B | double | Fixed | 728.83 | - | - | - | - |
| rate_IS_F_B2F_S_NB | double | Fixed | 430.43 | - | - | - | - |
| rate_IS_F_B2IS_F_NB | double | Fixed | 1680.0 | - | - | - | - |
| rate_IS_F_B2IS_S_B | double | Fixed | 1164.71 | - | - | - | - |
| rate_IS_F_B2IS_S_NB | double | Fixed | 0.0 | - | - | - | - |
| rate_IS_F_NB2F_F_B | double | Fixed | 963.55 | - | - | - | - |
| rate_IS_F_NB2F_F_NB | double | Fixed | 948.25 | - | - | - | - |
| rate_IS_F_NB2F_S_B | double | Fixed | 560.88 | - | - | - | - |

| Range Study Variable Assignments for Study range_st in Project AV_Intersection_with_Motorway b: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Variable | Type | Range Type | Range | Increment | Increment Type | Function | n |
| rate_IS_F_NB2F_S_NB | double | Fixed | 918.64 | - | - | - | - |
| rate_IS_F_NB2IS_F_B | double | Fixed | 1800.0 | - | - | - | - |
| rate_IS_F_NB2IS_S_B | double | Fixed | 936.29 | - | - | - | - |
| rate_IS_F_NB2IS_S_NB | double | Fixed | 1109.88 | - | - | - | - |
| rate_IS_S_B2F_F_B | double | Fixed | 0.0 | - | - | - | - |
| rate_IS_S_B2F_F_NB | double | Fixed | 573.8 | - | - | - | - |
| rate_IS_S_B2F_S_B | double | Fixed | 848.17 | - | - | - | - |
| rate_IS_S_B2F_S_NB | double | Fixed | 511.62 | - | - | - | - |
| rate_IS_S_B2IS_F_B | double | Fixed | 0.0 | - | - | - | - |
| rate_IS_S_B2IS_F_NB | double | Fixed | 743.98 | - | - | - | - |
| rate_IS_S_B2IS_S_NB | double | Fixed | 645.875 | - | - | - | - |
| rate_IS_S_NB2F_F_B | double | Fixed | 0.0 | - | - | - | - |
| rate_IS_S_NB2F_F_NB | double | Fixed | 207.38 | - | - | - | - |
| rate_IS_S_NB2F_S_B | double | Fixed | 328.93 | - | - | - | - |
| rate_IS_S_NB2F_S_NB | double | Fixed | 324.69 | - | - | - | - |
| rate_IS_S_NB2IS_F_B | double | Fixed | 0.0 | - | - | - | - |
| rate_IS_S_NB2IS_F_NB | double | Fixed | 678.65 | - | - | - | - |
| rate_IS_S_NB2IS_S_B | double | Fixed | 601.59 | - | - | - | - |

| Range Study Variable Assignments for Study range_st_v2 in Project AV_Intersection_with_Motorway a: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Variable | Type | Range Type | Range | Increment | Increment Type | Function | n |
| CH2AccRate_FFB | double | Fixed | 0.0856 | - | - | - | - |
| CH2AccRate_FSB | double | Fixed | 0.0856 | - | - | - | - |
| CH2AccRate_ISFB | double | Fixed | 0.0856 | - | - | - | - |
| CH2AccRate_ISSB | double | Fixed | 0.0856 | - | - | - | - |
| F_F_B_O_prob | double | Fixed | 1.0E-6 | - | - | - | - |
| F_S_B_O_prob | double | Fixed | 1.0E-6 | - | - | - | - |
| IS_F_B_O_prob | double | Fixed | 1.0E-6 | - | - | - | - |
| IS_S_B_O_prob | double | Fixed | 1.0E-6 | - | - | - | - |
| MH2FFB_AccRate | double | Fixed | 0.1 | - | - | - | - |
| MH2FSB_AccRate | double | Fixed | 0.1 | - | - | - | - |
| MH2ISFB_AccRate | double | Fixed | 0.1 | - | - | - | - |
| MH2ISSB_AccRate | double | Fixed | 0.1 | - | - | - | - |
| OH2CH_FFB_scale | double | Fixed | 10.0 | - | - | - | - |
| OH2CH_FSB_scale | double | Fixed | 10.0 | - | - | - | - |
| OH2CH_ISFB_scale | double | Fixed | 10.0 | - | - | - | - |
| OH2CH_ISSB_scale | double | Fixed | 10.0 | - | - | - | - |
| OH2OKRate_FFB | double | Fixed | 4500.0 | - | - | - | - |
| OH2OKRate_FSB | double | Fixed | 2250.0 | - | - | - | - |
| OH2OKRate_ISFB | double | Fixed | 2250.0 | - | - | - | - |
| OH2OKRate_ISSB | double | Fixed | 1225.0 | - | - | - | - |
| rate_F_F_B2F_F_NB | double | Fixed | 609.87 | - | - | - | - |
| rate_F_F_B2F_S_B | double | Fixed | 676.71 | - | - | - | - |
| rate_F_F_B2F_S_NB | double | Fixed | 319.02 | - | - | - | - |
| rate_F_F_B2IS_F_B | double | Fixed | 616.29 | - | - | - | - |
| rate_F_F_B2IS_F_NB | double | Fixed | 551.1 | - | - | - | - |
| rate_F_F_B2IS_S_B | double | Fixed | 549.09 | - | - | - | - |
| rate_F_F_B2_IS_S_NB | double | Fixed | 310.79 | - | - | - | - |
| rate_F_F_NB2F_F_B | double | Fixed | 170.61 | - | - | - | - |
| rate_F_F_NB2F_S_B | double | Fixed | 252.79 | - | - | - | - |
| rate_F_F_NB2F_S_NB | double | Fixed | 361.58 | - | - | - | - |
| rate_F_F_NB2IS_F_B | double | Fixed | 209.77 | - | - | - | - |
| rate_F_F_NB2IS_F_NB | double | Fixed | 202.345 | - | - | - | - |
| rate_F_F_NB2IS_S_B | double | Fixed | 361.87 | - | - | - | - |
| rate_F_F_NB2IS_S_NB | double | Fixed | 273.315 | - | - | - | - |
| rate_F_S_B2F_F_B | double | Fixed | 1080.0 | - | - | - | - |
| rate_F_S_B2F_F_NB | double | Fixed | 604.64 | - | - | - | - |
| rate_F_S_B2F_S_NB | double | Fixed | 465.98 | - | - | - | - |
| rate_F_S_B2IS_F_B | double | Fixed | 0.0 | - | - | - | - |
| rate_F_S_B2IS_F_NB | double | Fixed | 439.22 | - | - | - | - |
| rate_F_S_B2IS_S_B | double | Fixed | 940.01 | - | - | - | - |
| rate_F_S_B2IS_S_NB | double | Fixed | 438.63 | - | - | - | - |
| rate_F_S_NB2F_F_B | double | Fixed | 568.42 | - | - | - | - |
| rate_F_S_NB2F_F_NB | double | Fixed | 223.71 | - | - | - | - |
| rate_F_S_NB2F_S_B | double | Fixed | 355.8 | - | - | - | - |
| rate_F_S_NB2IS_F_B | double | Fixed | 0.0 | - | - | - | - |
| rate_F_S_NB2IS_F_NB | double | Fixed | 264.45 | - | - | - | - |
| rate_F_S_NB2IS_S_B | double | Fixed | 219.02 | - | - | - | - |
| rate_F_S_NB2IS_S_NB | double | Fixed | 228.71 | - | - | - | - |
| rate_IS_F_B2F_F_B | double | Fixed | 986.54 | - | - | - | - |
| rate_IS_F_B2F_F_NB | double | Fixed | 822.71 | - | - | - | - |
| rate_IS_F_B2F_S_B | double | Fixed | 728.83 | - | - | - | - |
| rate_IS_F_B2F_S_NB | double | Fixed | 430.43 | - | - | - | - |
| rate_IS_F_B2IS_F_NB | double | Fixed | 1680.0 | - | - | - | - |
| rate_IS_F_B2IS_S_B | double | Fixed | 1164.71 | - | - | - | - |
| rate_IS_F_B2IS_S_NB | double | Fixed | 0.0 | - | - | - | - |
| rate_IS_F_NB2F_F_B | double | Fixed | 963.55 | - | - | - | - |
| rate_IS_F_NB2F_F_NB | double | Fixed | 948.25 | - | - | - | - |
| rate_IS_F_NB2F_S_B | double | Fixed | 560.88 | - | - | - | - |

| Range Study Variable Assignments for Study range_st_v2 in Project AV_Intersection_with_Motorway b: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Variable | Type | Range Type | Range | Increment | Increment Type | Function | n |
| rate_IS_F_NB2F_S_NB | double | Fixed | 918.64 | - | - | - | - |
| rate_IS_F_NB2IS_F_B | double | Fixed | 1800.0 | - | - | - | - |
| rate_IS_F_NB2IS_S_B | double | Fixed | 936.29 | - | - | - | - |
| rate_IS_F_NB2IS_S_NB | double | Fixed | 1109.88 | - | - | - | - |
| rate_IS_S_B2F_F_B | double | Fixed | 0.0 | - | - | - | - |
| rate_IS_S_B2F_F_NB | double | Fixed | 573.8 | - | - | - | - |
| rate_IS_S_B2F_S_B | double | Fixed | 848.17 | - | - | - | - |
| rate_IS_S_B2F_S_NB | double | Fixed | 511.62 | - | - | - | - |
| rate_IS_S_B2IS_F_B | double | Fixed | 0.0 | - | - | - | - |
| rate_IS_S_B2IS_F_NB | double | Fixed | 743.98 | - | - | - | - |
| rate_IS_S_B2IS_S_NB | double | Fixed | 645.875 | - | - | - | - |
| rate_IS_S_NB2F_F_B | double | Fixed | 0.0 | - | - | - | - |
| rate_IS_S_NB2F_F_NB | double | Fixed | 207.38 | - | - | - | - |
| rate_IS_S_NB2F_S_B | double | Fixed | 328.93 | - | - | - | - |
| rate_IS_S_NB2F_S_NB | double | Fixed | 324.69 | - | - | - | - |
| rate_IS_S_NB2IS_F_B | double | Fixed | 0.0 | - | - | - | - |
| rate_IS_S_NB2IS_F_NB | double | Fixed | 678.65 | - | - | - | - |
| rate_IS_S_NB2IS_S_B | double | Fixed | 601.59 | - | - | - | - |

| Range Study Variable Assignments for Study range_st_v3 in Project AV_Intersection_with_Motorway a: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Variable | Type | Range Type | Range | Increment | Increment Type | Function | n |
| CH2AccRate_FFB | double | Fixed | 0.0856 | - | - | - | - |
| CH2AccRate_FSB | double | Fixed | 0.0856 | - | - | - | - |
| CH2AccRate_ISFB | double | Fixed | 0.0856 | - | - | - | - |
| CH2AccRate_ISSB | double | Fixed | 0.0856 | - | - | - | - |
| F_F_B_O_prob | double | Fixed | 1.0E-5 | - | - | - | - |
| F_S_B_O_prob | double | Fixed | 1.0E-5 | - | - | - | - |
| IS_F_B_O_prob | double | Fixed | 1.0E-5 | - | - | - | - |
| IS_S_B_O_prob | double | Fixed | 1.0E-5 | - | - | - | - |
| MH2FFB_AccRate | double | Fixed | 0.1 | - | - | - | - |
| MH2FSB_AccRate | double | Fixed | 0.1 | - | - | - | - |
| MH2ISFB_AccRate | double | Fixed | 0.1 | - | - | - | - |
| MH2ISSB_AccRate | double | Fixed | 0.1 | - | - | - | - |
| OH2CH_FFB_scale | double | Fixed | 10.0 | - | - | - | - |
| OH2CH_FSB_scale | double | Fixed | 10.0 | - | - | - | - |
| OH2CH_ISFB_scale | double | Fixed | 10.0 | - | - | - | - |
| OH2CH_ISSB_scale | double | Fixed | 10.0 | - | - | - | - |
| OH2OKRate_FFB | double | Fixed | 4500.0 | - | - | - | - |
| OH2OKRate_FSB | double | Fixed | 2250.0 | - | - | - | - |
| OH2OKRate_ISFB | double | Fixed | 2250.0 | - | - | - | - |
| OH2OKRate_ISSB | double | Fixed | 1225.0 | - | - | - | - |
| rate_F_F_B2F_F_NB | double | Fixed | 609.87 | - | - | - | - |
| rate_F_F_B2F_S_B | double | Fixed | 676.71 | - | - | - | - |
| rate_F_F_B2F_S_NB | double | Fixed | 319.02 | - | - | - | - |
| rate_F_F_B2IS_F_B | double | Fixed | 616.29 | - | - | - | - |
| rate_F_F_B2IS_F_NB | double | Fixed | 551.1 | - | - | - | - |
| rate_F_F_B2IS_S_B | double | Fixed | 549.09 | - | - | - | - |
| rate_F_F_B2_IS_S_NB | double | Fixed | 310.79 | - | - | - | - |
| rate_F_F_NB2F_F_B | double | Fixed | 170.61 | - | - | - | - |
| rate_F_F_NB2F_S_B | double | Fixed | 252.79 | - | - | - | - |
| rate_F_F_NB2F_S_NB | double | Fixed | 361.58 | - | - | - | - |
| rate_F_F_NB2IS_F_B | double | Fixed | 209.77 | - | - | - | - |
| rate_F_F_NB2IS_F_NB | double | Fixed | 202.345 | - | - | - | - |
| rate_F_F_NB2IS_S_B | double | Fixed | 361.87 | - | - | - | - |
| rate_F_F_NB2IS_S_NB | double | Fixed | 273.315 | - | - | - | - |
| rate_F_S_B2F_F_B | double | Fixed | 1080.0 | - | - | - | - |
| rate_F_S_B2F_F_NB | double | Fixed | 604.64 | - | - | - | - |
| rate_F_S_B2F_S_NB | double | Fixed | 465.98 | - | - | - | - |
| rate_F_S_B2IS_F_B | double | Fixed | 0.0 | - | - | - | - |
| rate_F_S_B2IS_F_NB | double | Fixed | 439.22 | - | - | - | - |
| rate_F_S_B2IS_S_B | double | Fixed | 940.01 | - | - | - | - |
| rate_F_S_B2IS_S_NB | double | Fixed | 438.63 | - | - | - | - |
| rate_F_S_NB2F_F_B | double | Fixed | 568.42 | - | - | - | - |
| rate_F_S_NB2F_F_NB | double | Fixed | 223.71 | - | - | - | - |
| rate_F_S_NB2F_S_B | double | Fixed | 355.8 | - | - | - | - |
| rate_F_S_NB2IS_F_B | double | Fixed | 0.0 | - | - | - | - |
| rate_F_S_NB2IS_F_NB | double | Fixed | 264.45 | - | - | - | - |
| rate_F_S_NB2IS_S_B | double | Fixed | 219.02 | - | - | - | - |
| rate_F_S_NB2IS_S_NB | double | Fixed | 228.71 | - | - | - | - |
| rate_IS_F_B2F_F_B | double | Fixed | 986.54 | - | - | - | - |
| rate_IS_F_B2F_F_NB | double | Fixed | 822.71 | - | - | - | - |
| rate_IS_F_B2F_S_B | double | Fixed | 728.83 | - | - | - | - |
| rate_IS_F_B2F_S_NB | double | Fixed | 430.43 | - | - | - | - |
| rate_IS_F_B2IS_F_NB | double | Fixed | 1680.0 | - | - | - | - |
| rate_IS_F_B2IS_S_B | double | Fixed | 1164.71 | - | - | - | - |
| rate_IS_F_B2IS_S_NB | double | Fixed | 0.0 | - | - | - | - |
| rate_IS_F_NB2F_F_B | double | Fixed | 963.55 | - | - | - | - |
| rate_IS_F_NB2F_F_NB | double | Fixed | 948.25 | - | - | - | - |
| rate_IS_F_NB2F_S_B | double | Fixed | 560.88 | - | - | - | - |

| Range Study Variable Assignments for Study range_st_v3 in Project AV_Intersection_with_Motorway b: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Variable | Type | Range Type | Range | Increment | Increment Type | Function | n |
| rate_IS_F_NB2F_S_NB | double | Fixed | 918.64 | - | - | - | - |
| rate_IS_F_NB2IS_F_B | double | Fixed | 1800.0 | - | - | - | - |
| rate_IS_F_NB2IS_S_B | double | Fixed | 936.29 | - | - | - | - |
| rate_IS_F_NB2IS_S_NB | double | Fixed | 1109.88 | - | - | - | - |
| rate_IS_S_B2F_F_B | double | Fixed | 0.0 | - | - | - | - |
| rate_IS_S_B2F_F_NB | double | Fixed | 573.8 | - | - | - | - |
| rate_IS_S_B2F_S_B | double | Fixed | 848.17 | - | - | - | - |
| rate_IS_S_B2F_S_NB | double | Fixed | 511.62 | - | - | - | - |
| rate_IS_S_B2IS_F_B | double | Fixed | 0.0 | - | - | - | - |
| rate_IS_S_B2IS_F_NB | double | Fixed | 743.98 | - | - | - | - |
| rate_IS_S_B2IS_S_NB | double | Fixed | 645.875 | - | - | - | - |
| rate_IS_S_NB2F_F_B | double | Fixed | 0.0 | - | - | - | - |
| rate_IS_S_NB2F_F_NB | double | Fixed | 207.38 | - | - | - | - |
| rate_IS_S_NB2F_S_B | double | Fixed | 328.93 | - | - | - | - |
| rate_IS_S_NB2F_S_NB | double | Fixed | 324.69 | - | - | - | - |
| rate_IS_S_NB2IS_F_B | double | Fixed | 0.0 | - | - | - | - |
| rate_IS_S_NB2IS_F_NB | double | Fixed | 678.65 | - | - | - | - |
| rate_IS_S_NB2IS_S_B | double | Fixed | 601.59 | - | - | - | - |

| Set Study: fixed_sta: |  |  |  |
| --- | --- | --- | --- |
| Experiment | Variable | Type | Value |
| Experiment 1 | F_F_B_O_prob | double | 0 |
|  | F_S_B_O_prob | double | 0 |
|  | HDLateAcc_FFB_prob | double | 0 |
|  | HDLateAcc_FSB_prob | double | 0 |
|  | HDLateAcc_ISFB_prob | double | 0 |
|  | HDLateAcc_ISSB_prob | double | 0 |
|  | IS_F_B_O_prob | double | 0 |
|  | IS_S_B_O_prob | double | 0 |
|  | MisHazFailFFB_prob | double | 0 |
|  | MisHazFailFSB_prob | double | 0 |
|  | MisHazFailISFB_prob | double | 0 |
|  | MisHazFailISSB_prob | double | 0 |
|  | OH2OKRate_FFB | double | 4500.0 |
|  | OH2OKRate_FSB | double | 2250.0 |
|  | OH2OKRate_ISFB | double | 2250.0 |
|  | OH2OKRate_ISSB | double | 1225.0 |
|  | SafMonFailFSB_prob | double | 0.0 |
|  | SafMonFailISFB_prob | double | 0.0 |
|  | SafMonFailISSB_prob | double | 0.0 |
|  | SafMonFail_FFB_prob | double | 0.0 |
|  | rate_F_F_B2F_F_NB | double | 609.87 |
|  | rate_F_F_B2F_S_B | double | 676.71 |
|  | rate_F_F_B2F_S_NB | double | 319.02 |
|  | rate_F_F_B2IS_F_B | double | 616.29 |
|  | rate_F_F_B2IS_F_NB | double | 551.1 |
|  | rate_F_F_B2IS_S_B | double | 549.09 |
|  | rate_F_F_B2_IS_S_NB | double | 310.79 |
|  | rate_F_F_NB2F_F_B | double | 170.61 |
|  | rate_F_F_NB2F_S_B | double | 252.79 |
|  | rate_F_F_NB2F_S_NB | double | 361.58 |
|  | rate_F_F_NB2IS_F_B | double | 209.77 |
|  | rate_F_F_NB2IS_F_NB | double | 202.345 |
|  | rate_F_F_NB2IS_S_B | double | 361.87 |
|  | rate_F_F_NB2IS_S_NB | double | 273.315 |
|  | rate_F_S_B2F_F_B | double | 1080.0 |
|  | rate_F_S_B2F_F_NB | double | 604.64 |
|  | rate_F_S_B2F_S_NB | double | 465.98 |
|  | rate_F_S_B2IS_F_B | double | 0.0 |
|  | rate_F_S_B2IS_F_NB | double | 439.22 |
|  | rate_F_S_B2IS_S_B | double | 940.01 |
|  | rate_F_S_B2IS_S_NB | double | 438.63 |
|  | rate_F_S_NB2F_F_B | double | 568.63 |
|  | rate_F_S_NB2F_F_NB | double | 223.71 |
|  | rate_F_S_NB2F_S_B | double | 355.8 |
|  | rate_F_S_NB2IS_F_B | double | 0.0 |
|  | rate_F_S_NB2IS_F_NB | double | 264.45 |
|  | rate_F_S_NB2IS_S_B | double | 219.02 |
|  | rate_F_S_NB2IS_S_NB | double | 228.71 |
|  | rate_IS_F_B2F_F_B | double | 986.54 |
|  | rate_IS_F_B2F_F_NB | double | 822.71 |
|  | rate_IS_F_B2F_S_B | double | 728.83 |
|  | rate_IS_F_B2F_S_NB | double | 430.43 |
|  | rate_IS_F_B2IS_F_NB | double | 1680.0 |
|  | rate_IS_F_B2IS_S_B | double | 1164.71 |
|  | rate_IS_F_B2IS_S_NB | double | 0.0 |
|  | rate_IS_F_NB2F_F_B | double | 963.55 |
|  | rate_IS_F_NB2F_F_NB | double | 948.25 |
|  | rate_IS_F_NB2F_S_B | double | 560.88 |

| Set Study: fixed_stb: |  |  |  |
| --- | --- | --- | --- |
| Experiment | Variable | Type | Value |
| Experiment 1 | rate_IS_F_NB2F_S_NB | double | 918.64 |
|  | rate_IS_F_NB2IS_F_B | double | 1800.0 |
|  | rate_IS_F_NB2IS_S_B | double | 936.29 |
|  | rate_IS_F_NB2IS_S_NB | double | 1109.88 |
|  | rate_IS_S_B2F_F_B | double | 0.0 |
|  | rate_IS_S_B2F_F_NB | double | 573.8 |
|  | rate_IS_S_B2F_S_B | double | 848.17 |
|  | rate_IS_S_B2F_S_NB | double | 511.62 |
|  | rate_IS_S_B2IS_F_B | double | 0.0 |
|  | rate_IS_S_B2IS_F_NB | double | 743.98 |
|  | rate_IS_S_B2IS_S_NB | double | 645.875 |
|  | rate_IS_S_NB2F_F_B | double | 0.0 |
|  | rate_IS_S_NB2F_F_NB | double | 207.38 |
|  | rate_IS_S_NB2F_S_B | double | 328.93 |
|  | rate_IS_S_NB2F_S_NB | double | 324.69 |
|  | rate_IS_S_NB2IS_F_B | double | 0.0 |
|  | rate_IS_S_NB2IS_F_NB | double | 678.65 |
|  | rate_IS_S_NB2IS_S_B | double | 601.59 |
| Experiment 2 | F_F_B_O_prob | double | 1.0E-6 |
|  | F_S_B_O_prob | double | 1.0E-6 |
|  | HDLateAcc_FFB_prob | double | 1.0E-4 |
|  | HDLateAcc_FSB_prob | double | 1.0E-4 |
|  | HDLateAcc_ISFB_prob | double | 1.0E-4 |
|  | HDLateAcc_ISSB_prob | double | 1.0E-4 |
|  | IS_F_B_O_prob | double | 1.0E-6 |
|  | IS_S_B_O_prob | double | 1.0E-6 |
|  | MisHazFailFFB_prob | double | 1.0E-6 |
|  | MisHazFailFSB_prob | double | 1.0E-6 |
|  | MisHazFailISFB_prob | double | 1.0E-6 |
|  | MisHazFailISSB_prob | double | 1.0E-6 |
|  | OH2OKRate_FFB | double | 4500.0 |
|  | OH2OKRate_FSB | double | 2250.0 |
|  | OH2OKRate_ISFB | double | 2250.0 |
|  | OH2OKRate_ISSB | double | 1225.0 |
|  | SafMonFailFSB_prob | double | 1.0E-6 |
|  | SafMonFailISFB_prob | double | 1.0E-6 |
|  | SafMonFailISSB_prob | double | 1.0E-6 |
|  | SafMonFail_FFB_prob | double | 1.0E-6 |
|  | rate_F_F_B2F_F_NB | double | 609.87 |
|  | rate_F_F_B2F_S_B | double | 676.71 |
|  | rate_F_F_B2F_S_NB | double | 319.02 |
|  | rate_F_F_B2IS_F_B | double | 616.29 |
|  | rate_F_F_B2IS_F_NB | double | 551.1 |
|  | rate_F_F_B2IS_S_B | double | 549.09 |
|  | rate_F_F_B2_IS_S_NB | double | 310.79 |
|  | rate_F_F_NB2F_F_B | double | 170.61 |
|  | rate_F_F_NB2F_S_B | double | 252.79 |
|  | rate_F_F_NB2F_S_NB | double | 361.58 |
|  | rate_F_F_NB2IS_F_B | double | 209.77 |
|  | rate_F_F_NB2IS_F_NB | double | 202.345 |
|  | rate_F_F_NB2IS_S_B | double | 361.87 |
|  | rate_F_F_NB2IS_S_NB | double | 273.315 |
|  | rate_F_S_B2F_F_B | double | 1080.0 |
|  | rate_F_S_B2F_F_NB | double | 604.64 |
|  | rate_F_S_B2F_S_NB | double | 465.98 |
|  | rate_F_S_B2IS_F_B | double | 0.0 |
|  | rate_F_S_B2IS_F_NB | double | 439.22 |

| Set Study: fixed_stc: |  |  |  |
| --- | --- | --- | --- |
| Experiment | Variable | Type | Value |
| Experiment 2 | rate_F_S_B2IS_S_B | double | 940.01 |
|  | rate_F_S_B2IS_S_NB | double | 438.63 |
|  | rate_F_S_NB2F_F_B | double | 568.63 |
|  | rate_F_S_NB2F_F_NB | double | 223.71 |
|  | rate_F_S_NB2F_S_B | double | 355.8 |
|  | rate_F_S_NB2IS_F_B | double | 0.0 |
|  | rate_F_S_NB2IS_F_NB | double | 264.45 |
|  | rate_F_S_NB2IS_S_B | double | 219.02 |
|  | rate_F_S_NB2IS_S_NB | double | 228.71 |
|  | rate_IS_F_B2F_F_B | double | 986.54 |
|  | rate_IS_F_B2F_F_NB | double | 822.71 |
|  | rate_IS_F_B2F_S_B | double | 728.83 |
|  | rate_IS_F_B2F_S_NB | double | 430.43 |
|  | rate_IS_F_B2IS_F_NB | double | 1680.0 |
|  | rate_IS_F_B2IS_S_B | double | 1164.71 |
|  | rate_IS_F_B2IS_S_NB | double | 0.0 |
|  | rate_IS_F_NB2F_F_B | double | 963.55 |
|  | rate_IS_F_NB2F_F_NB | double | 948.25 |
|  | rate_IS_F_NB2F_S_B | double | 560.88 |
|  | rate_IS_F_NB2F_S_NB | double | 918.64 |
|  | rate_IS_F_NB2IS_F_B | double | 1800.0 |
|  | rate_IS_F_NB2IS_S_B | double | 936.29 |
|  | rate_IS_F_NB2IS_S_NB | double | 1109.88 |
|  | rate_IS_S_B2F_F_B | double | 0.0 |
|  | rate_IS_S_B2F_F_NB | double | 573.8 |
|  | rate_IS_S_B2F_S_B | double | 848.17 |
|  | rate_IS_S_B2F_S_NB | double | 511.62 |
|  | rate_IS_S_B2IS_F_B | double | 0.0 |
|  | rate_IS_S_B2IS_F_NB | double | 743.98 |
|  | rate_IS_S_B2IS_S_NB | double | 645.875 |
|  | rate_IS_S_NB2F_F_B | double | 0.0 |
|  | rate_IS_S_NB2F_F_NB | double | 207.38 |
|  | rate_IS_S_NB2F_S_B | double | 328.93 |
|  | rate_IS_S_NB2F_S_NB | double | 324.69 |
|  | rate_IS_S_NB2IS_F_B | double | 0.0 |
|  | rate_IS_S_NB2IS_F_NB | double | 678.65 |
|  | rate_IS_S_NB2IS_S_B | double | 601.59 |
| Experiment 3 | F_F_B_O_prob | double | 1.0E-5 |
|  | F_S_B_O_prob | double | 1.0E-5 |
|  | HDLateAcc_FFB_prob | double | 1.0E-4 |
|  | HDLateAcc_FSB_prob | double | 1.0E-4 |
|  | HDLateAcc_ISFB_prob | double | 1.0E-4 |
|  | HDLateAcc_ISSB_prob | double | 1.0E-4 |
|  | IS_F_B_O_prob | double | 1.0E-5 |
|  | IS_S_B_O_prob | double | 1.0E-5 |
|  | MisHazFailFFB_prob | double | 1.0E-5 |
|  | MisHazFailFSB_prob | double | 1.0E-5 |
|  | MisHazFailISFB_prob | double | 1.0E-5 |
|  | MisHazFailISSB_prob | double | 1.0E-5 |
|  | OH2OKRate_FFB | double | 4500.0 |
|  | OH2OKRate_FSB | double | 2250.0 |
|  | OH2OKRate_ISFB | double | 2250.0 |
|  | OH2OKRate_ISSB | double | 1225.0 |
|  | SafMonFailFSB_prob | double | 1.0E-5 |
|  | SafMonFailISFB_prob | double | 1.0E-5 |
|  | SafMonFailISSB_prob | double | 1.0E-5 |
|  | SafMonFail_FFB_prob | double | 1.0E-5 |

| Set Study: fixed_std: |  |  |  |
| --- | --- | --- | --- |
| Experiment | Variable | Type | Value |
| Experiment 3 | rate_F_F_B2F_F_NB | double | 609.87 |
|  | rate_F_F_B2F_S_B | double | 676.71 |
|  | rate_F_F_B2F_S_NB | double | 319.02 |
|  | rate_F_F_B2IS_F_B | double | 616.29 |
|  | rate_F_F_B2IS_F_NB | double | 551.1 |
|  | rate_F_F_B2IS_S_B | double | 549.09 |
|  | rate_F_F_B2_IS_S_NB | double | 310.79 |
|  | rate_F_F_NB2F_F_B | double | 170.61 |
|  | rate_F_F_NB2F_S_B | double | 252.79 |
|  | rate_F_F_NB2F_S_NB | double | 361.58 |
|  | rate_F_F_NB2IS_F_B | double | 209.77 |
|  | rate_F_F_NB2IS_F_NB | double | 202.345 |
|  | rate_F_F_NB2IS_S_B | double | 361.87 |
|  | rate_F_F_NB2IS_S_NB | double | 273.315 |
|  | rate_F_S_B2F_F_B | double | 1080.0 |
|  | rate_F_S_B2F_F_NB | double | 604.64 |
|  | rate_F_S_B2F_S_NB | double | 465.98 |
|  | rate_F_S_B2IS_F_B | double | 0.0 |
|  | rate_F_S_B2IS_F_NB | double | 439.22 |
|  | rate_F_S_B2IS_S_B | double | 940.01 |
|  | rate_F_S_B2IS_S_NB | double | 438.63 |
|  | rate_F_S_NB2F_F_B | double | 568.63 |
|  | rate_F_S_NB2F_F_NB | double | 223.71 |
|  | rate_F_S_NB2F_S_B | double | 355.8 |
|  | rate_F_S_NB2IS_F_B | double | 0.0 |
|  | rate_F_S_NB2IS_F_NB | double | 264.45 |
|  | rate_F_S_NB2IS_S_B | double | 219.02 |
|  | rate_F_S_NB2IS_S_NB | double | 228.71 |
|  | rate_IS_F_B2F_F_B | double | 986.54 |
|  | rate_IS_F_B2F_F_NB | double | 822.71 |
|  | rate_IS_F_B2F_S_B | double | 728.83 |
|  | rate_IS_F_B2F_S_NB | double | 430.43 |
|  | rate_IS_F_B2IS_F_NB | double | 1680.0 |
|  | rate_IS_F_B2IS_S_B | double | 1164.71 |
|  | rate_IS_F_B2IS_S_NB | double | 0.0 |
|  | rate_IS_F_NB2F_F_B | double | 963.55 |
|  | rate_IS_F_NB2F_F_NB | double | 948.25 |
|  | rate_IS_F_NB2F_S_B | double | 560.88 |
|  | rate_IS_F_NB2F_S_NB | double | 918.64 |
|  | rate_IS_F_NB2IS_F_B | double | 1800.0 |
|  | rate_IS_F_NB2IS_S_B | double | 936.29 |
|  | rate_IS_F_NB2IS_S_NB | double | 1109.88 |
|  | rate_IS_S_B2F_F_B | double | 0.0 |
|  | rate_IS_S_B2F_F_NB | double | 573.8 |
|  | rate_IS_S_B2F_S_B | double | 848.17 |
|  | rate_IS_S_B2F_S_NB | double | 511.62 |
|  | rate_IS_S_B2IS_F_B | double | 0.0 |
|  | rate_IS_S_B2IS_F_NB | double | 743.98 |
|  | rate_IS_S_B2IS_S_NB | double | 645.875 |
|  | rate_IS_S_NB2F_F_B | double | 0.0 |
|  | rate_IS_S_NB2F_F_NB | double | 207.38 |
|  | rate_IS_S_NB2F_S_B | double | 328.93 |
|  | rate_IS_S_NB2F_S_NB | double | 324.69 |
|  | rate_IS_S_NB2IS_F_B | double | 0.0 |
|  | rate_IS_S_NB2IS_F_NB | double | 678.65 |
|  | rate_IS_S_NB2IS_S_B | double | 601.59 |
| Experiment 4 | F_F_B_O_prob | double | 1.0E-4 |

| Set Study: fixed_ste: |  |  |  |
| --- | --- | --- | --- |
| Experiment | Variable | Type | Value |
| Experiment 4 | F_S_B_O_prob | double | 1.0E-4 |
|  | HDLateAcc_FFB_prob | double | 0.001 |
|  | HDLateAcc_FSB_prob | double | 0.001 |
|  | HDLateAcc_ISFB_prob | double | 0.001 |
|  | HDLateAcc_ISSB_prob | double | 0.001 |
|  | IS_F_B_O_prob | double | 1.0E-4 |
|  | IS_S_B_O_prob | double | 1.0E-4 |
|  | MisHazFailFFB_prob | double | 1.0E-4 |
|  | MisHazFailFSB_prob | double | 1.0E-4 |
|  | MisHazFailISFB_prob | double | 1.0E-4 |
|  | MisHazFailISSB_prob | double | 1.0E-4 |
|  | OH2OKRate_FFB | double | 4500.0 |
|  | OH2OKRate_FSB | double | 2250.0 |
|  | OH2OKRate_ISFB | double | 2250.0 |
|  | OH2OKRate_ISSB | double | 1225.0 |
|  | SafMonFailFSB_prob | double | 1.0E-4 |
|  | SafMonFailISFB_prob | double | 1.0E-4 |
|  | SafMonFailISSB_prob | double | 1.0E-4 |
|  | SafMonFail_FFB_prob | double | 1.0E-4 |
|  | rate_F_F_B2F_F_NB | double | 609.87 |
|  | rate_F_F_B2F_S_B | double | 676.71 |
|  | rate_F_F_B2F_S_NB | double | 319.02 |
|  | rate_F_F_B2IS_F_B | double | 616.29 |
|  | rate_F_F_B2IS_F_NB | double | 551.1 |
|  | rate_F_F_B2IS_S_B | double | 549.09 |
|  | rate_F_F_B2_IS_S_NB | double | 310.79 |
|  | rate_F_F_NB2F_F_B | double | 170.61 |
|  | rate_F_F_NB2F_S_B | double | 252.79 |
|  | rate_F_F_NB2F_S_NB | double | 361.58 |
|  | rate_F_F_NB2IS_F_B | double | 209.77 |
|  | rate_F_F_NB2IS_F_NB | double | 202.345 |
|  | rate_F_F_NB2IS_S_B | double | 361.87 |
|  | rate_F_F_NB2IS_S_NB | double | 273.315 |
|  | rate_F_S_B2F_F_B | double | 1080.0 |
|  | rate_F_S_B2F_F_NB | double | 604.64 |
|  | rate_F_S_B2F_S_NB | double | 465.98 |
|  | rate_F_S_B2IS_F_B | double | 0.0 |
|  | rate_F_S_B2IS_F_NB | double | 439.22 |
|  | rate_F_S_B2IS_S_B | double | 940.01 |
|  | rate_F_S_B2IS_S_NB | double | 438.63 |
|  | rate_F_S_NB2F_F_B | double | 568.63 |
|  | rate_F_S_NB2F_F_NB | double | 223.71 |
|  | rate_F_S_NB2F_S_B | double | 355.8 |
|  | rate_F_S_NB2IS_F_B | double | 0.0 |
|  | rate_F_S_NB2IS_F_NB | double | 264.45 |
|  | rate_F_S_NB2IS_S_B | double | 219.02 |
|  | rate_F_S_NB2IS_S_NB | double | 228.71 |
|  | rate_IS_F_B2F_F_B | double | 986.54 |
|  | rate_IS_F_B2F_F_NB | double | 822.71 |
|  | rate_IS_F_B2F_S_B | double | 728.83 |
|  | rate_IS_F_B2F_S_NB | double | 430.43 |
|  | rate_IS_F_B2IS_F_NB | double | 1680.0 |
|  | rate_IS_F_B2IS_S_B | double | 1164.71 |
|  | rate_IS_F_B2IS_S_NB | double | 0.0 |
|  | rate_IS_F_NB2F_F_B | double | 963.55 |
|  | rate_IS_F_NB2F_F_NB | double | 948.25 |
|  | rate_IS_F_NB2F_S_B | double | 560.88 |
|  | rate_IS_F_NB2F_S_NB | double | 918.64 |

| Set Study: fixed_stf: |  |  |  |
| --- | --- | --- | --- |
| Experiment | Variable | Type | Value |
| Experiment 4 | rate_IS_F_NB2IS_F_B | double | 1800.0 |
|  | rate_IS_F_NB2IS_S_B | double | 936.29 |
|  | rate_IS_F_NB2IS_S_NB | double | 1109.88 |
|  | rate_IS_S_B2F_F_B | double | 0.0 |
|  | rate_IS_S_B2F_F_NB | double | 573.8 |
|  | rate_IS_S_B2F_S_B | double | 848.17 |
|  | rate_IS_S_B2F_S_NB | double | 511.62 |
|  | rate_IS_S_B2IS_F_B | double | 0.0 |
|  | rate_IS_S_B2IS_F_NB | double | 743.98 |
|  | rate_IS_S_B2IS_S_NB | double | 645.875 |
|  | rate_IS_S_NB2F_F_B | double | 0.0 |
|  | rate_IS_S_NB2F_F_NB | double | 207.38 |
|  | rate_IS_S_NB2F_S_B | double | 328.93 |
|  | rate_IS_S_NB2F_S_NB | double | 324.69 |
|  | rate_IS_S_NB2IS_F_B | double | 0.0 |
|  | rate_IS_S_NB2IS_F_NB | double | 678.65 |
|  | rate_IS_S_NB2IS_S_B | double | 601.59 |

# Appendix A: Baseline case with no perception failures

Mobius trs Version 2.4,

Copyright (c) William H. Sanders and Univ. of Illinois 1994-2010

******************************************************************************

Mobius TRANSIENT SOLVER

TRANSIENT SOLVER RESULTS

******************************************************************************

Started: Wed Aug 31 11:32:45 2022

Project name: AV_Intersection_with_Motorway

Study name: fixed_st

State space name: sspg_fx

Experiment name: Experiment_1

Global variable settings for this experiment:

F_F_B_O_prob = 0

F_S_B_O_prob = 0

HDLateAcc_FFB_prob = 0

HDLateAcc_FSB_prob = 0

HDLateAcc_ISFB_prob = 0

HDLateAcc_ISSB_prob = 0

IS_F_B_O_prob = 0

IS_S_B_O_prob = 0

MisHazFailFFB_prob = 0

MisHazFailFSB_prob = 0

MisHazFailISFB_prob = 0

MisHazFailISSB_prob = 0

OH2OKRate_FFB = 4500.0

OH2OKRate_FSB = 2250.0

OH2OKRate_ISFB = 2250.0

OH2OKRate_ISSB = 1225.0

SafMonFailFSB_prob = 0.0

SafMonFailISFB_prob = 0.0

SafMonFailISSB_prob = 0.0

SafMonFail_FFB_prob = 0.0

rate_F_F_B2F_F_NB = 609.87

rate_F_F_B2F_S_B = 676.71

rate_F_F_B2F_S_NB = 319.02

rate_F_F_B2IS_F_B = 616.29

rate_F_F_B2IS_F_NB = 551.1

rate_F_F_B2IS_S_B = 549.09

rate_F_F_B2_IS_S_NB = 310.79

rate_F_F_NB2F_F_B = 170.61

rate_F_F_NB2F_S_B = 252.79

rate_F_F_NB2F_S_NB = 361.58

rate_F_F_NB2IS_F_B = 209.77

rate_F_F_NB2IS_F_NB = 202.345

rate_F_F_NB2IS_S_B = 361.87

rate_F_F_NB2IS_S_NB = 273.315

rate_F_S_B2F_F_B = 1080.0

rate_F_S_B2F_F_NB = 604.64

rate_F_S_B2F_S_NB = 465.98

rate_F_S_B2IS_F_B = 0.0

rate_F_S_B2IS_F_NB = 439.22

rate_F_S_B2IS_S_B = 940.01

rate_F_S_B2IS_S_NB = 438.63

rate_F_S_NB2F_F_B = 568.63

rate_F_S_NB2F_F_NB = 223.71

rate_F_S_NB2F_S_B = 355.8

rate_F_S_NB2IS_F_B = 0.0

rate_F_S_NB2IS_F_NB = 264.45

rate_F_S_NB2IS_S_B = 219.02

rate_F_S_NB2IS_S_NB = 228.71

rate_IS_F_B2F_F_B = 986.54

rate_IS_F_B2F_F_NB = 822.71

rate_IS_F_B2F_S_B = 728.83

rate_IS_F_B2F_S_NB = 430.43

rate_IS_F_B2IS_F_NB = 1680.0

rate_IS_F_B2IS_S_B = 1164.71

rate_IS_F_B2IS_S_NB = 0.0

rate_IS_F_NB2F_F_B = 963.55

rate_IS_F_NB2F_F_NB = 948.25

rate_IS_F_NB2F_S_B = 560.88

rate_IS_F_NB2F_S_NB = 918.64

rate_IS_F_NB2IS_F_B = 1800.0

rate_IS_F_NB2IS_S_B = 936.29

rate_IS_F_NB2IS_S_NB = 1109.88

rate_IS_S_B2F_F_B = 0.0

rate_IS_S_B2F_F_NB = 573.8

rate_IS_S_B2F_S_B = 848.17

rate_IS_S_B2F_S_NB = 511.62

rate_IS_S_B2IS_F_B = 0.0

rate_IS_S_B2IS_F_NB = 743.98

rate_IS_S_B2IS_S_NB = 645.875

rate_IS_S_NB2F_F_B = 0.0

rate_IS_S_NB2F_F_NB = 207.38

rate_IS_S_NB2F_S_B = 328.93

rate_IS_S_NB2F_S_NB = 324.69

rate_IS_S_NB2IS_F_B = 0.0

rate_IS_S_NB2IS_F_NB = 678.65

rate_IS_S_NB2IS_S_B = 601.59

Verbosity: 0

Accuracy: 1.000000e-009

Time points: 100.000000 1100.000000 2100.000000 3100.000000 4100.000000 5100.000000 6100.000000 7100.000000 8100.000000 9100.000000

Computation Time (seconds): 1340.186000

User Time (seconds): 1340.186000

System Time (seconds): 0.000000

Rate of Poisson process: 7237.490000

Number of states in process: 10

Number of states with reward: 0

Number of time points: 10

Time Point       Left Trunc.      # Iterations          Error

---------------   ---------------   ---------------   ---------------

100.000000            717792            732173     1.000000e-009

1100.000000           7941486           7989173     1.000000e-009

2100.000000          15171437          15237325     1.000000e-009

3100.000000          22403060          22483112     1.000000e-009

4100.000000          29635575          29727637     1.000000e-009

5100.000000          36868669          36971345     1.000000e-009

6100.000000          44102176          44214468     1.000000e-009

7100.000000          51335998          51457145     1.000000e-009

8100.000000          58570071          58699468     1.000000e-009

9100.000000          65804349          65941500     1.000000e-009

******************************************************************************

Performance variable :  TimeToAccident

Time                 :  100.000000

Mean                 :  0.000000e+000

Variance             :  0.000000e+000

Plot files    (pdf)  :  Experiment_1.trs.TimeToAccident.100.000.pdf.splot

(cdf)  :  Experiment_1.trs.TimeToAccident.100.000.cdf.splot

******************************************************************************

Performance variable :  TimeToAccident

Time                 :  1100.000000

Mean                 :  0.000000e+000

Variance             :  0.000000e+000

Plot files    (pdf)  :  Experiment_1.trs.TimeToAccident.1100.000.pdf.splot

(cdf)  :  Experiment_1.trs.TimeToAccident.1100.000.cdf.splot

******************************************************************************

Performance variable :  TimeToAccident

Time                 :  2100.000000

Mean                 :  0.000000e+000

Variance             :  0.000000e+000

Plot files    (pdf)  :  Experiment_1.trs.TimeToAccident.2100.000.pdf.splot

(cdf)  :  Experiment_1.trs.TimeToAccident.2100.000.cdf.splot

******************************************************************************

Performance variable :  TimeToAccident

Time                 :  3100.000000

Mean                 :  0.000000e+000

Variance             :  0.000000e+000

Plot files    (pdf)  :  Experiment_1.trs.TimeToAccident.3100.000.pdf.splot

(cdf)  :  Experiment_1.trs.TimeToAccident.3100.000.cdf.splot

******************************************************************************

Performance variable :  TimeToAccident

Time                 :  4100.000000

Mean                 :  0.000000e+000

Variance             :  0.000000e+000

Plot files    (pdf)  :  Experiment_1.trs.TimeToAccident.4100.000.pdf.splot

(cdf)  :  Experiment_1.trs.TimeToAccident.4100.000.cdf.splot

******************************************************************************

Performance variable :  TimeToAccident

Time                 :  5100.000000

Mean                 :  0.000000e+000

Variance             :  0.000000e+000

Plot files    (pdf)  :  Experiment_1.trs.TimeToAccident.5100.000.pdf.splot

(cdf)  :  Experiment_1.trs.TimeToAccident.5100.000.cdf.splot

******************************************************************************

Performance variable :  TimeToAccident

Time                 :  6100.000000

Mean                 :  0.000000e+000

Variance             :  0.000000e+000

Plot files    (pdf)  :  Experiment_1.trs.TimeToAccident.6100.000.pdf.splot

(cdf)  :  Experiment_1.trs.TimeToAccident.6100.000.cdf.splot

******************************************************************************

Performance variable :  TimeToAccident

Time                 :  7100.000000

Mean                 :  0.000000e+000

Variance             :  0.000000e+000

Plot files    (pdf)  :  Experiment_1.trs.TimeToAccident.7100.000.pdf.splot

(cdf)  :  Experiment_1.trs.TimeToAccident.7100.000.cdf.splot

******************************************************************************

Performance variable :  TimeToAccident

Time                 :  8100.000000

Mean                 :  0.000000e+000

Variance             :  0.000000e+000

Plot files    (pdf)  :  Experiment_1.trs.TimeToAccident.8100.000.pdf.splot

(cdf)  :  Experiment_1.trs.TimeToAccident.8100.000.cdf.splot

******************************************************************************

Performance variable :  TimeToAccident

Time                 :  9100.000000

Mean                 :  0.000000e+000

Variance             :  0.000000e+000

Plot files    (pdf)  :  Experiment_1.trs.TimeToAccident.9100.000.pdf.splot

(cdf)  :  Experiment_1.trs.TimeToAccident.9100.000.cdf.splot

******************************************************************************

Finished: Wed Aug 31 11:55:05 2022

# Appendix B

Mobius trs Version 2.4,

Copyright (c) William H. Sanders and Univ. of Illinois 1994-2010

******************************************************************************

Mobius TRANSIENT SOLVER

TRANSIENT SOLVER RESULTS

******************************************************************************

Started: Wed Aug 31 11:55:06 2022

Project name: AV_Intersection_with_Motorway

Study name: fixed_st

State space name: sspg_fx

Experiment name: Experiment_2

Global variable settings for this experiment:

F_F_B_O_prob = 1.0E-6

F_S_B_O_prob = 1.0E-6

HDLateAcc_FFB_prob = 1.0E-4

HDLateAcc_FSB_prob = 1.0E-4

HDLateAcc_ISFB_prob = 1.0E-4

HDLateAcc_ISSB_prob = 1.0E-4

IS_F_B_O_prob = 1.0E-6

IS_S_B_O_prob = 1.0E-6

MisHazFailFFB_prob = 1.0E-6

MisHazFailFSB_prob = 1.0E-6

MisHazFailISFB_prob = 1.0E-6

MisHazFailISSB_prob = 1.0E-6

OH2OKRate_FFB = 4500.0

OH2OKRate_FSB = 2250.0

OH2OKRate_ISFB = 2250.0

OH2OKRate_ISSB = 1225.0

SafMonFailFSB_prob = 1.0E-6

SafMonFailISFB_prob = 1.0E-6

SafMonFailISSB_prob = 1.0E-6

SafMonFail_FFB_prob = 1.0E-6

rate_F_F_B2F_F_NB = 609.87

rate_F_F_B2F_S_B = 676.71

rate_F_F_B2F_S_NB = 319.02

rate_F_F_B2IS_F_B = 616.29

rate_F_F_B2IS_F_NB = 551.1

rate_F_F_B2IS_S_B = 549.09

rate_F_F_B2_IS_S_NB = 310.79

rate_F_F_NB2F_F_B = 170.61

rate_F_F_NB2F_S_B = 252.79

rate_F_F_NB2F_S_NB = 361.58

rate_F_F_NB2IS_F_B = 209.77

rate_F_F_NB2IS_F_NB = 202.345

rate_F_F_NB2IS_S_B = 361.87

rate_F_F_NB2IS_S_NB = 273.315

rate_F_S_B2F_F_B = 1080.0

rate_F_S_B2F_F_NB = 604.64

rate_F_S_B2F_S_NB = 465.98

rate_F_S_B2IS_F_B = 0.0

rate_F_S_B2IS_F_NB = 439.22

rate_F_S_B2IS_S_B = 940.01

rate_F_S_B2IS_S_NB = 438.63

rate_F_S_NB2F_F_B = 568.63

rate_F_S_NB2F_F_NB = 223.71

rate_F_S_NB2F_S_B = 355.8

rate_F_S_NB2IS_F_B = 0.0

rate_F_S_NB2IS_F_NB = 264.45

rate_F_S_NB2IS_S_B = 219.02

rate_F_S_NB2IS_S_NB = 228.71

rate_IS_F_B2F_F_B = 986.54

rate_IS_F_B2F_F_NB = 822.71

rate_IS_F_B2F_S_B = 728.83

rate_IS_F_B2F_S_NB = 430.43

rate_IS_F_B2IS_F_NB = 1680.0

rate_IS_F_B2IS_S_B = 1164.71

rate_IS_F_B2IS_S_NB = 0.0

rate_IS_F_NB2F_F_B = 963.55

rate_IS_F_NB2F_F_NB = 948.25

rate_IS_F_NB2F_S_B = 560.88

rate_IS_F_NB2F_S_NB = 918.64

rate_IS_F_NB2IS_F_B = 1800.0

rate_IS_F_NB2IS_S_B = 936.29

rate_IS_F_NB2IS_S_NB = 1109.88

rate_IS_S_B2F_F_B = 0.0

rate_IS_S_B2F_F_NB = 573.8

rate_IS_S_B2F_S_B = 848.17

rate_IS_S_B2F_S_NB = 511.62

rate_IS_S_B2IS_F_B = 0.0

rate_IS_S_B2IS_F_NB = 743.98

rate_IS_S_B2IS_S_NB = 645.875

rate_IS_S_NB2F_F_B = 0.0

rate_IS_S_NB2F_F_NB = 207.38

rate_IS_S_NB2F_S_B = 328.93

rate_IS_S_NB2F_S_NB = 324.69

rate_IS_S_NB2IS_F_B = 0.0

rate_IS_S_NB2IS_F_NB = 678.65

rate_IS_S_NB2IS_S_B = 601.59

Verbosity: 0

Accuracy: 1.000000e-009

Time points: 100.000000 1100.000000 2100.000000 3100.000000 4100.000000 5100.000000 6100.000000 7100.000000 8100.000000 9100.000000

Computation Time (seconds): 1774.234000

User Time (seconds): 1774.234000

System Time (seconds): 0.000000

Rate of Poisson process: 7237.490000

Number of states in process: 17

Number of states with reward: 1

Number of time points: 10

Time Point       Left Trunc.      # Iterations          Error

---------------   ---------------   ---------------   ---------------

100.000000            717792            732173     1.000000e-009

1100.000000           7941486           7989173     1.000000e-009

2100.000000          15171437          15237325     1.000000e-009

3100.000000          22403060          22483112     1.000000e-009

4100.000000          29635575          29727637     1.000000e-009

5100.000000          36868669          36971345     1.000000e-009

6100.000000          44102176          44214468     1.000000e-009

7100.000000          51335998          51457145     1.000000e-009

8100.000000          58570071          58699468     1.000000e-009

9100.000000          65804349          65941500     1.000000e-009

******************************************************************************

Performance variable :  TimeToAccident

Time                 :  100.000000

Mean                 :  2.651354e-005

Variance             :  2.651283e-005

Plot files    (pdf)  :  Experiment_2.trs.TimeToAccident.100.000.pdf.splot

(cdf)  :  Experiment_2.trs.TimeToAccident.100.000.cdf.splot

******************************************************************************

Performance variable :  TimeToAccident

Time                 :  1100.000000

Mean                 :  2.651354e-005

Variance             :  2.651283e-005

Plot files    (pdf)  :  Experiment_2.trs.TimeToAccident.1100.000.pdf.splot

(cdf)  :  Experiment_2.trs.TimeToAccident.1100.000.cdf.splot

******************************************************************************

Performance variable :  TimeToAccident

Time                 :  2100.000000

Mean                 :  2.651354e-005

Variance             :  2.651283e-005

Plot files    (pdf)  :  Experiment_2.trs.TimeToAccident.2100.000.pdf.splot

(cdf)  :  Experiment_2.trs.TimeToAccident.2100.000.cdf.splot

******************************************************************************

Performance variable :  TimeToAccident

Time                 :  3100.000000

Mean                 :  2.651354e-005

Variance             :  2.651283e-005

Plot files    (pdf)  :  Experiment_2.trs.TimeToAccident.3100.000.pdf.splot

(cdf)  :  Experiment_2.trs.TimeToAccident.3100.000.cdf.splot

******************************************************************************

Performance variable :  TimeToAccident

Time                 :  4100.000000

Mean                 :  2.651354e-005

Variance             :  2.651283e-005

Plot files    (pdf)  :  Experiment_2.trs.TimeToAccident.4100.000.pdf.splot

(cdf)  :  Experiment_2.trs.TimeToAccident.4100.000.cdf.splot

******************************************************************************

Performance variable :  TimeToAccident

Time                 :  5100.000000

Mean                 :  2.651354e-005

Variance             :  2.651283e-005

Plot files    (pdf)  :  Experiment_2.trs.TimeToAccident.5100.000.pdf.splot

(cdf)  :  Experiment_2.trs.TimeToAccident.5100.000.cdf.splot

******************************************************************************

Performance variable :  TimeToAccident

Time                 :  6100.000000

Mean                 :  2.651354e-005

Variance             :  2.651283e-005

Plot files    (pdf)  :  Experiment_2.trs.TimeToAccident.6100.000.pdf.splot

(cdf)  :  Experiment_2.trs.TimeToAccident.6100.000.cdf.splot

******************************************************************************

Performance variable :  TimeToAccident

Time                 :  7100.000000

Mean                 :  2.651354e-005

Variance             :  2.651283e-005

Plot files    (pdf)  :  Experiment_2.trs.TimeToAccident.7100.000.pdf.splot

(cdf)  :  Experiment_2.trs.TimeToAccident.7100.000.cdf.splot

******************************************************************************

Performance variable :  TimeToAccident

Time                 :  8100.000000

Mean                 :  2.651354e-005

Variance             :  2.651283e-005

Plot files    (pdf)  :  Experiment_2.trs.TimeToAccident.8100.000.pdf.splot

(cdf)  :  Experiment_2.trs.TimeToAccident.8100.000.cdf.splot

******************************************************************************

Performance variable :  TimeToAccident

Time                 :  9100.000000

Mean                 :  2.651354e-005

Variance             :  2.651283e-005

Plot files    (pdf)  :  Experiment_2.trs.TimeToAccident.9100.000.pdf.splot

(cdf)  :  Experiment_2.trs.TimeToAccident.9100.000.cdf.splot

******************************************************************************

Finished: Wed Aug 31 12:24:40 2022

# Appendix C

Mobius trs Version 2.4,

Copyright (c) William H. Sanders and Univ. of Illinois 1994-2010

******************************************************************************

Mobius TRANSIENT SOLVER

TRANSIENT SOLVER RESULTS

******************************************************************************

Started: Wed Aug 31 12:24:40 2022

Project name: AV_Intersection_with_Motorway

Study name: fixed_st

State space name: sspg_fx

Experiment name: Experiment_3

Global variable settings for this experiment:

F_F_B_O_prob = 1.0E-5

F_S_B_O_prob = 1.0E-5

HDLateAcc_FFB_prob = 1.0E-4

HDLateAcc_FSB_prob = 1.0E-4

HDLateAcc_ISFB_prob = 1.0E-4

HDLateAcc_ISSB_prob = 1.0E-4

IS_F_B_O_prob = 1.0E-5

IS_S_B_O_prob = 1.0E-5

MisHazFailFFB_prob = 1.0E-5

MisHazFailFSB_prob = 1.0E-5

MisHazFailISFB_prob = 1.0E-5

MisHazFailISSB_prob = 1.0E-5

OH2OKRate_FFB = 4500.0

OH2OKRate_FSB = 2250.0

OH2OKRate_ISFB = 2250.0

OH2OKRate_ISSB = 1225.0

SafMonFailFSB_prob = 1.0E-5

SafMonFailISFB_prob = 1.0E-5

SafMonFailISSB_prob = 1.0E-5

SafMonFail_FFB_prob = 1.0E-5

rate_F_F_B2F_F_NB = 609.87

rate_F_F_B2F_S_B = 676.71

rate_F_F_B2F_S_NB = 319.02

rate_F_F_B2IS_F_B = 616.29

rate_F_F_B2IS_F_NB = 551.1

rate_F_F_B2IS_S_B = 549.09

rate_F_F_B2_IS_S_NB = 310.79

rate_F_F_NB2F_F_B = 170.61

rate_F_F_NB2F_S_B = 252.79

rate_F_F_NB2F_S_NB = 361.58

rate_F_F_NB2IS_F_B = 209.77

rate_F_F_NB2IS_F_NB = 202.345

rate_F_F_NB2IS_S_B = 361.87

rate_F_F_NB2IS_S_NB = 273.315

rate_F_S_B2F_F_B = 1080.0

rate_F_S_B2F_F_NB = 604.64

rate_F_S_B2F_S_NB = 465.98

rate_F_S_B2IS_F_B = 0.0

rate_F_S_B2IS_F_NB = 439.22

rate_F_S_B2IS_S_B = 940.01

rate_F_S_B2IS_S_NB = 438.63

rate_F_S_NB2F_F_B = 568.63

rate_F_S_NB2F_F_NB = 223.71

rate_F_S_NB2F_S_B = 355.8

rate_F_S_NB2IS_F_B = 0.0

rate_F_S_NB2IS_F_NB = 264.45

rate_F_S_NB2IS_S_B = 219.02

rate_F_S_NB2IS_S_NB = 228.71

rate_IS_F_B2F_F_B = 986.54

rate_IS_F_B2F_F_NB = 822.71

rate_IS_F_B2F_S_B = 728.83

rate_IS_F_B2F_S_NB = 430.43

rate_IS_F_B2IS_F_NB = 1680.0

rate_IS_F_B2IS_S_B = 1164.71

rate_IS_F_B2IS_S_NB = 0.0

rate_IS_F_NB2F_F_B = 963.55

rate_IS_F_NB2F_F_NB = 948.25

rate_IS_F_NB2F_S_B = 560.88

rate_IS_F_NB2F_S_NB = 918.64

rate_IS_F_NB2IS_F_B = 1800.0

rate_IS_F_NB2IS_S_B = 936.29

rate_IS_F_NB2IS_S_NB = 1109.88

rate_IS_S_B2F_F_B = 0.0

rate_IS_S_B2F_F_NB = 573.8

rate_IS_S_B2F_S_B = 848.17

rate_IS_S_B2F_S_NB = 511.62

rate_IS_S_B2IS_F_B = 0.0

rate_IS_S_B2IS_F_NB = 743.98

rate_IS_S_B2IS_S_NB = 645.875

rate_IS_S_NB2F_F_B = 0.0

rate_IS_S_NB2F_F_NB = 207.38

rate_IS_S_NB2F_S_B = 328.93

rate_IS_S_NB2F_S_NB = 324.69

rate_IS_S_NB2IS_F_B = 0.0

rate_IS_S_NB2IS_F_NB = 678.65

rate_IS_S_NB2IS_S_B = 601.59

Verbosity: 0

Accuracy: 1.000000e-009

Time points: 100.000000 1100.000000 2100.000000 3100.000000 4100.000000 5100.000000 6100.000000 7100.000000 8100.000000 9100.000000

Computation Time (seconds): 1727.085000

User Time (seconds): 1727.085000

System Time (seconds): 0.000000

Rate of Poisson process: 7237.490000

Number of states in process: 17

Number of states with reward: 1

Number of time points: 10

Time Point       Left Trunc.      # Iterations          Error

---------------   ---------------   ---------------   ---------------

100.000000            717792            732173     1.000000e-009

1100.000000           7941486           7989173     1.000000e-009

2100.000000          15171437          15237325     1.000000e-009

3100.000000          22403060          22483112     1.000000e-009

4100.000000          29635575          29727637     1.000000e-009

5100.000000          36868669          36971345     1.000000e-009

6100.000000          44102176          44214468     1.000000e-009

7100.000000          51335998          51457145     1.000000e-009

8100.000000          58570071          58699468     1.000000e-009

9100.000000          65804349          65941500     1.000000e-009

******************************************************************************

Performance variable :  TimeToAccident

Time                 :  100.000000

Mean                 :  2.650754e-004

Variance             :  2.650051e-004

Plot files    (pdf)  :  Experiment_3.trs.TimeToAccident.100.000.pdf.splot

(cdf)  :  Experiment_3.trs.TimeToAccident.100.000.cdf.splot

******************************************************************************

Performance variable :  TimeToAccident

Time                 :  1100.000000

Mean                 :  2.650754e-004

Variance             :  2.650051e-004

Plot files    (pdf)  :  Experiment_3.trs.TimeToAccident.1100.000.pdf.splot

(cdf)  :  Experiment_3.trs.TimeToAccident.1100.000.cdf.splot

******************************************************************************

Performance variable :  TimeToAccident

Time                 :  2100.000000

Mean                 :  2.650754e-004

Variance             :  2.650051e-004

Plot files    (pdf)  :  Experiment_3.trs.TimeToAccident.2100.000.pdf.splot

(cdf)  :  Experiment_3.trs.TimeToAccident.2100.000.cdf.splot

******************************************************************************

Performance variable :  TimeToAccident

Time                 :  3100.000000

Mean                 :  2.650754e-004

Variance             :  2.650051e-004

Plot files    (pdf)  :  Experiment_3.trs.TimeToAccident.3100.000.pdf.splot

(cdf)  :  Experiment_3.trs.TimeToAccident.3100.000.cdf.splot

******************************************************************************

Performance variable :  TimeToAccident

Time                 :  4100.000000

Mean                 :  2.650754e-004

Variance             :  2.650051e-004

Plot files    (pdf)  :  Experiment_3.trs.TimeToAccident.4100.000.pdf.splot

(cdf)  :  Experiment_3.trs.TimeToAccident.4100.000.cdf.splot

******************************************************************************

Performance variable :  TimeToAccident

Time                 :  5100.000000

Mean                 :  2.650754e-004

Variance             :  2.650051e-004

Plot files    (pdf)  :  Experiment_3.trs.TimeToAccident.5100.000.pdf.splot

(cdf)  :  Experiment_3.trs.TimeToAccident.5100.000.cdf.splot

******************************************************************************

Performance variable :  TimeToAccident

Time                 :  6100.000000

Mean                 :  2.650754e-004

Variance             :  2.650051e-004

Plot files    (pdf)  :  Experiment_3.trs.TimeToAccident.6100.000.pdf.splot

(cdf)  :  Experiment_3.trs.TimeToAccident.6100.000.cdf.splot

******************************************************************************

Performance variable :  TimeToAccident

Time                 :  7100.000000

Mean                 :  2.650754e-004

Variance             :  2.650051e-004

Plot files    (pdf)  :  Experiment_3.trs.TimeToAccident.7100.000.pdf.splot

(cdf)  :  Experiment_3.trs.TimeToAccident.7100.000.cdf.splot

******************************************************************************

Performance variable :  TimeToAccident

Time                 :  8100.000000

Mean                 :  2.650754e-004

Variance             :  2.650051e-004

Plot files    (pdf)  :  Experiment_3.trs.TimeToAccident.8100.000.pdf.splot

(cdf)  :  Experiment_3.trs.TimeToAccident.8100.000.cdf.splot

******************************************************************************

Performance variable :  TimeToAccident

Time                 :  9100.000000

Mean                 :  2.650754e-004

Variance             :  2.650051e-004

Plot files    (pdf)  :  Experiment_3.trs.TimeToAccident.9100.000.pdf.splot

(cdf)  :  Experiment_3.trs.TimeToAccident.9100.000.cdf.splot

******************************************************************************

Finished: Wed Aug 31 12:53:27 2022

# Appendix D

Mobius trs Version 2.4,

Copyright (c) William H. Sanders and Univ. of Illinois 1994-2010

******************************************************************************

Mobius TRANSIENT SOLVER

TRANSIENT SOLVER RESULTS

******************************************************************************

Started: Wed Aug 31 12:53:28 2022

Project name: AV_Intersection_with_Motorway

Study name: fixed_st

State space name: sspg_fx

Experiment name: Experiment_4

Global variable settings for this experiment:

F_F_B_O_prob = 1.0E-4

F_S_B_O_prob = 1.0E-4

HDLateAcc_FFB_prob = 0.001

HDLateAcc_FSB_prob = 0.001

HDLateAcc_ISFB_prob = 0.001

HDLateAcc_ISSB_prob = 0.001

IS_F_B_O_prob = 1.0E-4

IS_S_B_O_prob = 1.0E-4

MisHazFailFFB_prob = 1.0E-4

MisHazFailFSB_prob = 1.0E-4

MisHazFailISFB_prob = 1.0E-4

MisHazFailISSB_prob = 1.0E-4

OH2OKRate_FFB = 4500.0

OH2OKRate_FSB = 2250.0

OH2OKRate_ISFB = 2250.0

OH2OKRate_ISSB = 1225.0

SafMonFailFSB_prob = 1.0E-4

SafMonFailISFB_prob = 1.0E-4

SafMonFailISSB_prob = 1.0E-4

SafMonFail_FFB_prob = 1.0E-4

rate_F_F_B2F_F_NB = 609.87

rate_F_F_B2F_S_B = 676.71

rate_F_F_B2F_S_NB = 319.02

rate_F_F_B2IS_F_B = 616.29

rate_F_F_B2IS_F_NB = 551.1

rate_F_F_B2IS_S_B = 549.09

rate_F_F_B2_IS_S_NB = 310.79

rate_F_F_NB2F_F_B = 170.61

rate_F_F_NB2F_S_B = 252.79

rate_F_F_NB2F_S_NB = 361.58

rate_F_F_NB2IS_F_B = 209.77

rate_F_F_NB2IS_F_NB = 202.345

rate_F_F_NB2IS_S_B = 361.87

rate_F_F_NB2IS_S_NB = 273.315

rate_F_S_B2F_F_B = 1080.0

rate_F_S_B2F_F_NB = 604.64

rate_F_S_B2F_S_NB = 465.98

rate_F_S_B2IS_F_B = 0.0

rate_F_S_B2IS_F_NB = 439.22

rate_F_S_B2IS_S_B = 940.01

rate_F_S_B2IS_S_NB = 438.63

rate_F_S_NB2F_F_B = 568.63

rate_F_S_NB2F_F_NB = 223.71

rate_F_S_NB2F_S_B = 355.8

rate_F_S_NB2IS_F_B = 0.0

rate_F_S_NB2IS_F_NB = 264.45

rate_F_S_NB2IS_S_B = 219.02

rate_F_S_NB2IS_S_NB = 228.71

rate_IS_F_B2F_F_B = 986.54

rate_IS_F_B2F_F_NB = 822.71

rate_IS_F_B2F_S_B = 728.83

rate_IS_F_B2F_S_NB = 430.43

rate_IS_F_B2IS_F_NB = 1680.0

rate_IS_F_B2IS_S_B = 1164.71

rate_IS_F_B2IS_S_NB = 0.0

rate_IS_F_NB2F_F_B = 963.55

rate_IS_F_NB2F_F_NB = 948.25

rate_IS_F_NB2F_S_B = 560.88

rate_IS_F_NB2F_S_NB = 918.64

rate_IS_F_NB2IS_F_B = 1800.0

rate_IS_F_NB2IS_S_B = 936.29

rate_IS_F_NB2IS_S_NB = 1109.88

rate_IS_S_B2F_F_B = 0.0

rate_IS_S_B2F_F_NB = 573.8

rate_IS_S_B2F_S_B = 848.17

rate_IS_S_B2F_S_NB = 511.62

rate_IS_S_B2IS_F_B = 0.0

rate_IS_S_B2IS_F_NB = 743.98

rate_IS_S_B2IS_S_NB = 645.875

rate_IS_S_NB2F_F_B = 0.0

rate_IS_S_NB2F_F_NB = 207.38

rate_IS_S_NB2F_S_B = 328.93

rate_IS_S_NB2F_S_NB = 324.69

rate_IS_S_NB2IS_F_B = 0.0

rate_IS_S_NB2IS_F_NB = 678.65

rate_IS_S_NB2IS_S_B = 601.59

Verbosity: 0

Accuracy: 1.000000e-009

Time points: 100.000000 1100.000000 2100.000000 3100.000000 4100.000000 5100.000000 6100.000000 7100.000000 8100.000000 9100.000000

Computation Time (seconds): 1801.252000

User Time (seconds): 1801.252000

System Time (seconds): 0.000000

Rate of Poisson process: 7237.490000

Number of states in process: 17

Number of states with reward: 1

Number of time points: 10

Time Point       Left Trunc.      # Iterations          Error

---------------   ---------------   ---------------   ---------------

100.000000            717792            732173     1.000000e-009

1100.000000           7941486           7989173     1.000000e-009

2100.000000          15171437          15237325     1.000000e-009

3100.000000          22403060          22483112     1.000000e-009

4100.000000          29635575          29727637     1.000000e-009

5100.000000          36868669          36971345     1.000000e-009

6100.000000          44102176          44214468     1.000000e-009

7100.000000          51335998          51457145     1.000000e-009

8100.000000          58570071          58699468     1.000000e-009

9100.000000          65804349          65941500     1.000000e-009

******************************************************************************

Performance variable :  TimeToAccident

Time                 :  100.000000

Mean                 :  2.645838e-003

Variance             :  2.638837e-003

Plot files    (pdf)  :  Experiment_4.trs.TimeToAccident.100.000.pdf.splot

(cdf)  :  Experiment_4.trs.TimeToAccident.100.000.cdf.splot

******************************************************************************

Performance variable :  TimeToAccident

Time                 :  1100.000000

Mean                 :  2.645838e-003

Variance             :  2.638837e-003

Plot files    (pdf)  :  Experiment_4.trs.TimeToAccident.1100.000.pdf.splot

(cdf)  :  Experiment_4.trs.TimeToAccident.1100.000.cdf.splot

******************************************************************************

Performance variable :  TimeToAccident

Time                 :  2100.000000

Mean                 :  2.645838e-003

Variance             :  2.638837e-003

Plot files    (pdf)  :  Experiment_4.trs.TimeToAccident.2100.000.pdf.splot

(cdf)  :  Experiment_4.trs.TimeToAccident.2100.000.cdf.splot

******************************************************************************

Performance variable :  TimeToAccident

Time                 :  3100.000000

Mean                 :  2.645838e-003

Variance             :  2.638837e-003

Plot files    (pdf)  :  Experiment_4.trs.TimeToAccident.3100.000.pdf.splot

(cdf)  :  Experiment_4.trs.TimeToAccident.3100.000.cdf.splot

******************************************************************************

Performance variable :  TimeToAccident

Time                 :  4100.000000

Mean                 :  2.645838e-003

Variance             :  2.638837e-003

Plot files    (pdf)  :  Experiment_4.trs.TimeToAccident.4100.000.pdf.splot

(cdf)  :  Experiment_4.trs.TimeToAccident.4100.000.cdf.splot

******************************************************************************

Performance variable :  TimeToAccident

Time                 :  5100.000000

Mean                 :  2.645838e-003

Variance             :  2.638837e-003

Plot files    (pdf)  :  Experiment_4.trs.TimeToAccident.5100.000.pdf.splot

(cdf)  :  Experiment_4.trs.TimeToAccident.5100.000.cdf.splot

******************************************************************************

Performance variable :  TimeToAccident

Time                 :  6100.000000

Mean                 :  2.645838e-003

Variance             :  2.638837e-003

Plot files    (pdf)  :  Experiment_4.trs.TimeToAccident.6100.000.pdf.splot

(cdf)  :  Experiment_4.trs.TimeToAccident.6100.000.cdf.splot

******************************************************************************

Performance variable :  TimeToAccident

Time                 :  7100.000000

Mean                 :  2.645838e-003

Variance             :  2.638837e-003

Plot files    (pdf)  :  Experiment_4.trs.TimeToAccident.7100.000.pdf.splot

(cdf)  :  Experiment_4.trs.TimeToAccident.7100.000.cdf.splot

******************************************************************************

Performance variable :  TimeToAccident

Time                 :  8100.000000

Mean                 :  2.645838e-003

Variance             :  2.638837e-003

Plot files    (pdf)  :  Experiment_4.trs.TimeToAccident.8100.000.pdf.splot

(cdf)  :  Experiment_4.trs.TimeToAccident.8100.000.cdf.splot

******************************************************************************

Performance variable :  TimeToAccident

Time                 :  9100.000000

Mean                 :  2.645838e-003

Variance             :  2.638837e-003

Plot files    (pdf)  :  Experiment_4.trs.TimeToAccident.9100.000.pdf.splot

(cdf)  :  Experiment_4.trs.TimeToAccident.9100.000.cdf.splot

******************************************************************************

Finished: Wed Aug 31 13:23:29 2022

The dataset used for model parameterisation is … <Elaborate on the nature of the dataset. I suspect that this is a non-AV dataset>